SCRAMmer V37.2 - June,7th 1992

------------------------------

Copyright (c) by Giuliano C. Peritore

All Rights Reserved

Written by:

Giuliano C. Peritore

Via Amaseno, 6

04100 Latina

ITALY

Phone: (+39)773 491692

E-Mail: mc6040@mclink.it

ATTENTION!

----------

1) Turning on Page Detect mode on non SCRAM chips will  probably

break down the system.  You have to reboot.  You are advised.

2) Turning off Write Allocate will cause trouble.  You  have  to

reboot.  You are advised.

3) Causing a Timeout  when  Timeouts  are disabled will hang the

system.  You have to reboot.  You are advised.

4) Avoid playing with Refresh  Rate  gadgets.  It will cause you

a lot of trouble.  You are advised.

5) BERR timeout will be set when SCRAMmer is invoked if DSACK is

not requested.

6) Avoid disk activity while using SCRAMmer until you are pretty

much experienced.

7) The modification of system parameters allowed by SCRAMmer  is

not  well  accepted  from Commodore, because some A3000s have

problems with the Ramsey chip.  I made this program  to  have

an utility which allows to read and modify this parameters in

order to play with them and  find  if  I  can  set on special

modes and get a speed gain.  Since I had no  problems with my

A3000 I think that this is a good idea.  So if  you  want you

can try to get a speed improvement, but  you  are on your own

risk.  And remember: if Commodore says "Don't do it" there is

a valid reason.  You are advised.

DISCLAIMER

----------

The author  has no  responsibilities for incidents or damages

that may result  from the  correct  or  incorrect  use  of  this

program.  You are on your own risk.

NOTES

-----

Owing to the particular functions of this  program  an A3000,

with its 68030 CPU and custom chips is strictly required.  SCRAM

chips are recommended but not needed.  Remember that the manipu-

lation of special system parameters should be done  through  the

OS.  I've done it when possible.  This application  should  also

have a very well behavior under 2.04 (required) since  I  had no

problems with it and I coded it very carefully.

The application opens on the Workbench screen or on a default

Public Screen if it Hijacks Workbench.  The  default  monospaced

shell font will be used, falling on topaz 8 if it is too big.

Owing to the high number of gadgets the  application requires

at least a 640x256 screen  to work properly but don't be  afraid

to try it on a smaller screen or via the Shell.

IMPORTANT

---------

This program is in the Public Domain so it may be distributed

freely as long as this archive remains intact,  the  program  is

not modified, and that no profit is gained from its distribution

without written permission of the author.  If the reports of the

program are used anywhere please tell  that  they  are  SCRAMmer

reports and state the version.  Software  and Hardware companies

may include this program on their disks.

Donations (software, hardware, money, ecc.) are very welcomed

togheter with comments, suggestions, bugs, infos and so on.

INTRODUCTION

------------

The idea of writing SCRAMmer prompted suddendly while  I  was

following  a  debate on A3000 ram chips on MC-Link.  I  want  to

thank  Paolo Baliano, Stefano Iacus, Massimiliano Marras,   Bal-

dassarre Profeta and Andrea Suatoni for allowing this debate.

Since there are a lot of tricks to  get more performance from

an Amiga and that there are  a  lot  of  tools to do  this (VBR,

SetRamsey, Cpu, SetCPU, etc. )  I  decided  to  write  a  unique

testing  and  setting tool to allow  users to set crucial system

parameters and test their Ram to see improvements.

SCRAMmer is also a powerful tool to know much  more  about  your

wonderful Amiga 3000.

This program  is completely fruit of my mind since I get only

ideas from other programs, but I recoded EVERYTHING from scratch

adding many and many new features togheter with 2.04 support and

a consistent and font-sensitive user interface.

USING SCRAMMER

--------------

Note:

-----

This program digs deeply into your system and  it  is easy to

misunderstand its functions.  There's a lot to talk about.

Please read instructions carefully.

Running SCRAMmer V37.2 from the Shell:

--------------------------------------

SCRAMmer has a lot of arguments.  Type SCRAMmer ? to see  its

syntax, reported here for completeness:

SHOW/S,DATACACHE/S,NODATACACHE/S,DATABURST/S,NODATABURST/S,

DATAFREEZE/S,NODATAFREEZE/S,INSTCACHE/S,NOINSTCACHE/S,

INSTBURST/S,NOINSTBURST/S,INSTFREEZE/S,NOINSTFREEZE/S,

WRITEALLOCATE/S,NOWRITEALLOCATE/S,COPYBACK/S,NOCOPYBACK/S,

PAGEDETECT/S,NOPAGEDETECT/S,BURST/S,NOBURST/S,WRAP/S,NOWRAP/S,

REFRESH/K/N,NOREFRESH/S,VBRFAST/S,VBRCHIP/S,TIMEOUT/S,

NOTIMEOUT/S,BERRTIMEOUT/S,DSACKTIMEOUT/S,BANK/K/N,SCRAMTEST/S,

FASTTEST/S,CHIPTEST/S,DOTIMEOUT/S,GUI/S,PRINT/S,ABOUT/S

Here's a description:

SHOW            - Show a report of all system parameters

DATACACHE       - Enable Data Cache

NODATACACHE     - Disable Data Cache

DATABURST       - Enable Data Burst

NODATABURST     - Disable Data Burst

DATAFREEZE      - Freeze Data Cache

NODATAFREEZE    - Free Data Cache

INSTCACHE       - Enable Instruction Cache

NOINSTCACHE     - Disable Instruction Cache

INSTBURST       - Enable Instruction Burst

NOINSTBURST     - Disable Instruction Burst

INSTFREEZE      - Freeze Instruction Cache

NOINSTFREEZE    - Free Instruction Cache

WRITEALLOCATE   - Enable Write Allocate

NOWRITEALLOCATE - Disable Write Allocate

COPYBACK        - Enable copyback

NOCOPYBACK      - Disable copyback

PAGEDETECT      - Enable Page Detect Mode

NOPAGEDETECT    - Disable Page Detect Mode

BURST           - Enable Burst Mode

NOBURST         - Disable Burst Mode

WRAP            - Enable Wrap Mode

NOWRAP          - Disable Wrap Mode

REFRESH n       - Enable refresh (0-154,1-238,2-380,3-Off)

NOREFRESH       - Disable RAM refresh

VBRFAST         - Move VBR to FAST memory

VBRCHIP         - Move VBR to CHIP memory

TIMEOUT         - Enable bus Timeout

NOTIMEOUT       - Disable bus Timeout

BERRTIMEOUT     - Set BERR timeout

DSACKTIMEOUT    - Set DSACK timeout

BANK n          - Select RAM bank for tests (0-1-2-3)

SCRAMTEST       - Test if bank is SCRAM

FASTTEST        - Test FAST memory (current bank)

CHIPTEST        - Test CHIP memory

DOTIMEOUT       - Cause a bus timeout

GUI             - Use Graphical User Interface

PRINT           - Print report on printer

ABOUT           - Show info about the program

That's all.  See below for function-by-function explanation.

Running SCRAMmer V37.2 from the Workbench:

------------------------------------------

Just click on SCRAMmer icon.  This will show up the GUI.

Click on gadgets, press right mouse button or the  Help  key,

or click on closewindow to exit.

May be there are problems with 640x200 screens.  Try a bigger

screen !

Understanding SCRAMmer

----------------------

SCRAMmer allows you to show up a lot of system parameters and

to modify them.  It is very easy to play using the GUI or  using

the Shell interface.  But it is very important that  you  under-

stand what you are doing.  SCRAMmer is logically divided in some

regions,  each  regarding a specific class of parameters.  Let's

describe them all.

CPU & FPU

---------

The CPU (Central  Processing  Unit) and  FPU (Floating  Point

Unit) are the core of your computer, so it is important to  know

that your system recognized what CPU and FPU are installed.

SCRAMmer show this.  There  are  a  lot of adjustable parameters

regarding the CPU.  They are data and  instruction caches, which

speed  up  the  software  reducing  memory  accesses, the  Write

Allocation and the Copyback mode (provided also on the 68040).

For more info about these  parameters  please  consult  Motorola

manuals.

For the maximum  speed  you  should  enable  caches,  bursts,

copyback and (I suppose) disable Write Allocate.

Remember anyway that Write Allocate causes a lot of  problems

on standard A3000 when disabled, that Copyback may cause trouble

to bad written programs, togheter with caches.  You are advised.

RAMSEY

------

The Ramsey chip is the  memory  controller of the A3000.  The

program  will  show  which  type  of Ramsey is installed in your

computer and its settings.  You can enable or disable  the  Page

Detect Mode, which is a special mode that takes  advantage  from

the presence of SCRAMS  boosting  system  performance.  You  can

enable or disable the Burst Mode, which  allows  the CPU to fill

its cache very efficiently, or you can  enable  or  disable  the

Wrap mode, which depending  on  the  application  will  optimize

burst transfers.

SCRAMmer will show also  which  memory Chips are used in your

Amiga.  You  could  also  change  the  refresh  rate,  but it is

discouraged.

GARY

----

The Gary chip is the bus controller of  the  A3000.  SCRAMmer

will show which  Gary  is  installed  in  your  system  and  its

settings.  It shows  if  there  have  been  timeouts  or  if the

computer has just cold-booted or it has been resetted.

It  is  also  possible  to  enable/disable  the  bus  timeout

feature which terminates accesses to non existent memory without

hanging the system.  You can  decide  to  terminate  via BERR or

via DSACK.

Note that the Timeout/Boot flags are reset  every  time  they

are read so you can miss something.

Beware of disabling timeout.  Accesses to non existent memory

will hang the system.

VBR

---

The Vector Base Register is the interrupt table  of  the CPU.

Putting it in FAST ram will improve system  throughput  speeding

up interrupt driven operations such as serial  downloads, vblank

processing and so on.  It will not improve tests results because

interrupts are disabled during the test.

DENISE & AGNUS

--------------

These section show which Agnus and Denise chips are installed

in your A3000.  Useful for checking ECS and PAL/NTSC.

TESTS & MEMORY INFO

-------------------

The test section allows to perform some tests  whose  results

are shown in the Memory Info section.  The memory  info  section

will show (in microseconds) the time that  is needed to read and

write in Chip and Fast memory about 12MB per test.

These test are very perilous  for  time  critical  activities

such as serial download, disk activity and so on.  Beware !

The Bank button allows you to choose  on  which  Fast  Memory

bank (of the four of your A3000) you want to perform the test.

The SCRAM Test checks if your bank is  populated  with  SCRAM

memory.  Note  that  if you  haven't  SCRAMs  you  may  hang the

system  and  have  to  reboot because the test will corrupt FAST

memory if not SCRAM.  If there is no memory in that bank it will

be correctly reported.

The Fast Speed and Chip Speed perform a test on the speed  of

read and write operations  on  Fast ( selected bank )  and  Chip

memory.  See below for additional infos on the test.

If the Fast Speed is performend  on  non existent memory then

meaningless results are reported.  Changing  the  Bank will zero

previous results.

The Timeout gadget will cause a timeout to  check  if  it  is

correctly reported and processed.  It  causes  two  timeouts  by

reading  two longs at $08000000  and  $0c000000.  Note  that  if

timeout is disabled you'll hang the system.

Don't disable timeout if you plan to call the Fast Speed test

on an empty Fast Ram bank (which is meaningless).

About

-----

Pressing the right mouse button or the Help key will pop up a

small window with credits and informations about SCRAMmer.

Scrammer Uses

-------------

You can use SCRAMmer in your startup-sequence to  set  system

parameters  at  boot time, for example with "SCRAMmer  DATACACHE

DATABURST INSTCACHE INSTBURST PAGEDETECT WRAP BURST VBRFAST".

You can put SCRAMmer in your hard disk to call it via the  WB

or via the Shell when you need it.  You can use SCRAMmer to test

memory and systems in many configuration to have clearer ideas.

Good Luck !

FINAL NOTE

----------

You may ask why there are a lot of options very dangerous  in

this program.  Many of them aren't correctly supported  now  but

thay have been included because I always hope of  future  better

Amigas supporting all this.  For example I think that  who has a

68040 with Copyback enabled may safely turn of Write Allocate.

Other tools are useful  for  testing  purposes.  If  you  are

designing a new board and  you  want  investigate  if  it  works

correctly on the bus you  can  disable  timeout  and  call  your

board.  It system hangs then your board is wrong.  Moreover  the

SCRAM test is  important,  because you  can  get  benefits  from

SCRAMs, and during the SCRAM  test  is  impossible  to  preserve

eventual PAGE ram.  That's why you have to reboot.

THE SPEED TEST

--------------

The speed test disables interrupts and DMAs  and  performs  a

cycle reading and writing about 12 megabytes of data  in  memory

using  the  instruction cache if enabled.  The  read  and  write

operations  are  grouped in  groups  of  four  longword  aligned

longword accesses.  The test  reports the  microseconds  elapsed

during the operation (using the timer.device).

Note that this kind of test won't show benefits derived  from

VBR, Wrap, and possibly Burst.

Maybe this is a good test, maybe  it's not.  But you are sure

that it's not addomesticated. Other ideas will be well accepted.

PLANNED IMPROVEMENTS

--------------------

The following  features will  be  added  if  I receive enough

return and support:

- Iconification

- Public Screen support improved

- Improved Tests

- On-Line Help

- Chip routines will report chip numbers if unknown

- Keyboard equivalents for gadgets

REPORTS

-------

I'd like very much to receive your  reports.  Please  include

full SCRAMmer output and  the  complete  specification  of  your

system.  Please include your best  performace reports or as many

as you can.  The report should be formed by:

- Description of your system

- Showconfig output

- Multiple SCRAMmer output

delimited by appropriate separators.  See example below.

REPORTS FOLLOW

--------------

System:  A3000 B2 Rev 9/01

ChipRAM: 2MB TC514258AP-80 SCRAM DIP

FastRAM: 4MB TC514402Z -80 SCRAM ZIP

/*------------------------------------------------------------------*/

CHIPS:  CPU 68030/68882fpu/68030mmu, ECS PAL Agnus, ECS Denise

VERS:  Kickstart version 37.175, Exec version 37.132, Disk version 37.67

RAM:  Node type $a, attribute $105 (FAST), from $7c00000 to $7f7ffff

Node type $a, attribute $303 (CHIP), from $400 to $1fffff

BOARDS:

CBM Bridgeboard:   Prod=513/1($201/$1) (@$200000 512K)

/*------------------------------------------------------------------*/

SCRAMmer V37.2 - Copyright  1992 Giuliano C. Peritore

CPU & FPU

CPU:            68030  FPU:            68882

Data Cache:     On   Data Burst:     On   Data Freeze:    Off

Inst Cache:     On   Inst Burst:     On   Inst Freeze:    Off

Write Allocate: On   Copyback:       Off

VBR

At: ($7c3f6f8) FAST

RAMSEY

Chip:           Normal

Page Detect:    On   Burst:          On   Wrap:           On

Refresh Rate:   238 clocks

RAM Chips:      1 Meg by 4 Bit

GARY

Chip:           Normal

Boot:           Warm  Timeout:        On   Timeouts:       No

DENISE

Chip:           ECS

AGNUS

Chip:           ECS PAL 8372

TESTS REPORT

Bank:              0

Fast Read:    840000

Fast Write:   740001

Chip Read:   2880000

Chip Write:  1780000

/*------------------------------------------------------------------*/