�������������������������������������������������������������������ķ

�                                                                   � �

�                                                                   � �

�                           A M I s e t u p                         � �

�                                                                   � �

� Documentation                                                     � �

�                                                                   � �

� Copyright (c) Robert Muchsel 1992-1995                            � �

� All rights reserved.                                              � �

�������������������������������������������������������������������ͼ �

����������������������������������������������������������������������

AMIsetup is an external setup program for 386/486/Pentium PCs with

`HighFlex BIOS' or `WinBIOS' from AMI.

If you ever considered your built-in setup as incomprehensible,

boring, inconvenient or incomplete, you've been waiting for

AMIsetup.

AMIsetup can save your configuration on disk and restore it. You

can even change setup options missing in your BIOS! AMIsetup cracks

your password and prints your setup as a list. You can install IDE

hard disks automatically! Add comprehensive on-line help, Soundblaster

support and an on-line reference that explains many setup options.

You can even print a personal BIOS manual!

For diagnostic purposes, AMIsetup can on many machines measure the

DMA frequency (avoid burnout of the DMA chip due to overload) and

display the CMOS error flags. You can display the revision of your

keyboard controller, change setup registers manually (experts only)

and display the BIOS ID. Switch AMIsetup to high resolution video

mode (EGA/VGA) and use the mouse...

And once you are finished, AMIsetup can reboot your computer.

Professionals will like the batch mode of AMIsetup. Configure

identical computers in blitz speed!

Important

���������

��������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

�                                                                    �

�                         USE AT YOUR OWN RISK!                      �

�  This program is based on undocumented data structures.            �

�  Improper use, program errors or bad luck can cause hardware,      �

�  software and stored data of the computer be irreversibly damaged! �

�  The user is advised always to make adequate Back Ups of all       �

�  valuable files or Data. Because he can have no control over how   �

�  this program is used                                              �

�          THE AUTHOR ACCEPTS NO LIABILITY FOR ANY DAMAGE!           �

�                                                                    �

��������������������������������������������������������������������;

��������������������������������������������������������������������

No part of this documentation may be reproduced, transmitted,

transcribed, stored in any retrieval system, or translated into any

other language or computer language in whole or in part, in any form

or by any means, whether it be electronic, mechanical, magnetic,

optical, manual or otherwise, without prior written consent of the

author, Robert Muchsel.

The author disclaims all warranties as to this software, whether

express or implied, including without limitation any implied warranties

of merchantability, fitness for a particular purpose, functionality,

data integrity or protection, in so far as permitted by applicable

legislation.

All trademarks are property of their respective owners and appear for

identification purposes only.

Shareware

���������

�������������������������������������������������������������������Ŀ

�  Writing this program has taken lots of work. If you continue     �

�  using it after a reasonable trial period of 30 days, you have    �

�  to pay a small fee to the author.                                �

�                                                                   �

�  SHAREWARE is NOT FREE SOFTWARE. You can, however, test shareware �

�  before purchasing it.                                            �

�                                                                   �

�  For detailed information, see appendix A.                        �

���������������������������������������������������������������������

You are encouraged to copy and pass on the unregistered shareware

version of this program, if

- the program is distributed in its original form

(complete, unmodified, uninstalled)

- the distribution fee does not exceed the equivalent of DM 15.-.

It is forbidden to distribute the program bundled with books,

magazines, etc. (including `bookware'). To do this, you must obtain

a written permission from the author.

The information contained in the on-line reference for registered users

must not be copied, published or separated from the program. The author

has taken every care to ensure the technical correctness of the

information, but cannot accept any liability in case of any error.

If you have comments or find an error, please send them to the author!

Contents

��������

1   Caution!

2   Starting the program

3   Menus in detail

3.1 System menu

3.2 Diagnostic menu

3.3 Backup menu

3.4 Setup

1. Standard setup

2. Extended/chipset setup

3. Change register

4. Password

5. Load ROM setup defaults, load ROM power-on defaults

6. Print current setup

7. Print reference manual

8. Analyze BIOS ID

9. Save changes and quit

10. Exit without saving

3.5 Tools menu

3.6 Help menu

4   Key shortcuts

Appendix

A   Registration, limits of the shareware version,

addresses, upgrades, about the author

B   AMIsetup and OS/2, AMI BIOS and OS/2, Windows, Windows NT

C   Trouble shooting

1. Is AMIsetup compatible with your BIOS?

2. Frequent errors with compatible BIOS versions

3. Stopgap: -O

4. Additional trouble shooting options

D   Glossary

E   If the machine won't boot...

F   Switching to the English user interface

G   Soundblaster

H   EISA

I   Batch mode

J   Obtaining new or bug fixed program revisions

K   Tested BIOS revisions

What's new

Changes are described in a separate file, AMISETUP.NEW.

1  Caution!

�����������

If you are not familiar with setup options or if you don't know

what they do, please consider whether changing the setup options

is really necessary.

�������������������������������������������������������������������Ŀ

�  On many PCs, the DMA clock speed can be changed.                 �

�  This may work at first, but later the DMA chip may be destroyed, �

�  thus making the motherboard useless!                             �

�                                                                   �

�  Another critical point is the password option. Since you cannot  �

�  erase CMOS RAM on some main boards (see appendix E), it may      �

�  happen that because of - forgetfulness, BIOS errors, different   �

�  keyboard layout, etc. - you are irreversibly locked out of your  �

�  computer.                                                        �

���������������������������������������������������������������������

Before making ANY change, ask yourself:

- is it necessary?

- what are the consequences?

AMIsetup saves your configuration into a file after installation

and whenever you invoke the program. However, this works only if

you don't use AMIsetup on a write protected diskette.

�������������������������������������������������������������������Ŀ

�  N E V E R   C H A N G E   A   R U N N I N G   S Y S T E M  !!!   �

���������������������������������������������������������������������

WHEN DID YOU DO YOUR LAST COMPLETE BACKUP???

2  Starting the program

�����������������������

First of all, please read the manual and the file `AMISETUP.NEW'.

The program needs write access to the current directory. Diskettes

must not be write protected if file operations are to be used.

Normally, you start AMIsetup without additional command-line options.

There are exceptions though (see appendix C). You should read

appendix B if you want to use AMIsetup under OS/2.

If you are using an LCD screen, invoke AMIsetup by `AMISETUP -MONO'

(or execute `MODE MONO' or `MODE BW80' first).

Adaptec SCSI Controllers: Be sure to use the ASPI4DOS driver to avoid

possible problems with the memory management unit.

Virus scanners from Central Point Software: The memory resident

virus scanner from Central Point thinks there was a `TH TH' virus

in AMIsetup. The non-resident version and scanners from other vendors

will show you that this is not true.

If using AMIsetup for the first time, you have to choose the desired

language:

������������� Welcome / Willkommen ! �����������Ŀ

�                                                �

�  This is a bilingual version of AMIsetup.      �

�  For English help texts and user interface,    �

�  press [E].                                    �

�                                                �

�  Dies ist eine zweisprachige Version von       �

�  AMIsetup. Dr�cken Sie [D] f�r deutsche Hilfe- �

�  texte und Benutzeroberfl�che.                 �

�                                                �

�  [E] English version     [D] deutsche Version  �

�                                                �

����������������������������������������AMIsetup��

Should you decide on the German version, please continue reading in

the file `AMISETUP.DOK'.

Next you must to accept the warning message with a `Y':

������������������ C A U T I O N ���������������Ŀ

�                                                �

�              USE AT YOUR OWN RISK!             �

�                                                �

�  This program is based on undocumented data    �

�  structures. Hard- and software and stored     �

�  data could be damaged irrevocably!            �

�                                                �

� THE AUTHOR WILL NOT BE LIABLE FOR ANY DAMAGES! �

�                                                �

� [Y] accept and start program   [N] back to DOS �

�                                                �

����������������������������������������AMIsetup��

If you have set a password under `System|Options', you will be prompted

to enter it.

��������������� Password �������������ͻ

�                                      �

� AMIsetup is password protected.      �

� Please enter the current password.   �

�                                      �

�  Password    ****                    �

�                                      �

�                                      �

�          OK   �      Help  �         �

�        ��������     ��������         �

��������������������������������������ͼ

Next, a dialogue box containing program revision and registration

number (or `UNREGISTERED') appears. If you haven't yet paid for

AMIsetup and a registration number appears anyway, you've got an

(illegal) pirated copy. Please contact the author.

��[�]���������� Information �������������ͻ

�                                         �

�             AMIsetup v2.99              �

�         REGISTERED (#.........)         �

�                                         �

�  Copyright (c) 1992-95 Robert Muchsel   �

�           All rights reserved           �

�                                         �

�                                         �

�           Heap memory:  84080           �

�               PCI System                �

�                                         �

�            OK    �     Help   �         �

�          ���������    ���������         �

�����������������������������������������ͼ

Quick start

�����������

In most parts, AMIsetup should explain itself. All explanations

are included in the on-line help. However, please note the

warning messages and the license restrictions of the personal

BIOS manual.

3  Menus in detail

������������������

Using the mouse or pressing <F10>, you get to the main menu bar.

There, you have the following pull down menus at your disposal:

System  Diagnostic  Backup  Setup  Tools  Help

��������������������������������������������������

Pressing <F1> gets you help. The on-line help contains nearly all of

the manual.

Hint for mouse users: The right button is equivalent to <Esc>.

3.1 System menu

���������������

System

�������������������Ŀ

� Options...        � 3.1.1

� Video mode  Alt-V � 3.1.2

�������������������Ĵ

� Reboot...         � 3.1.3

� Exit        Alt-X � 3.1.4

���������������������

3.1.1 Options...

��[�]���������������������� Options ��������������������������Ŀ

�                                                              �

�  Preferences                                                 �

�   [ ] Auto start setup                  BIOS source... �     �

�   [X] Enable reboot command             ����������������     �

�   [X] Beep on error                      Password...   �     �

�   [ ] Reverse mouse buttons             ����������������     �

�   [ ] Use Soundblaster                                       �

�   [ ] German language                                        �

�                                                              �

�  Display setup options                                       �

�   [X] � Hidden options                       Save      �     �

�   [ ] � Deleted options                 ����������������     �

�                                             Cancel     �     �

�  Backup file path                       ����������������     �

�   C:\AMISETUP\                               Help      �     �

�                                         ����������������     �

����������������������������������������������������������������

Here you can customize the program to your own needs.

Auto start setup

Instead of selecting Setup manually each time using the menu,

`Setup' can also be started automatically.

Enable reboot command

If this box is blank, the Reboot function in the System menu is

disabled.

In addition, you will not be asked whether you want to reboot after

saving your setup.

Beep on error

If this option is checked, the program beeps if you enter incorrect

values in input boxes.

Reverse mouse buttons

You can reverse right and left mouse buttons (for left-handers or

southpaws).

Soundblaster

See appendix G.

German language

You can disable the English user interface and work with the German

one instead.

Display setup options

There are two types of hidden options in the ROM BIOS.

`�' shows that the option is hidden in the built-in BIOS setup.

The `�' options work correctly in most cases.

`�' shows that the option is hidden in the built-in BIOS setup and

that AMIsetup cannot find a textual description of the possible

settings. Often `�' options don't work.

You can enable and disable both types. If neither `�' nor `�' are

enabled, then AMIsetup shows the same set of options as the ROM

setup.

Backup file path

You can change the directory of the backup files.

BIOS source

The following explanation is very technical. If do not fully

understand it, please leave the BIOS source at `Protected Mode'.

AMIsetup must completely read your BIOS. Since parts of the BIOS

are frequemtly hidden by memory managers or the chip set itself

after the system has booted, AMIsetup offers three different

methods of accessing the BIOS:

Protected Mode

This mode is the default mode. AMIsetup switches to protected mode

and thereby circumvents memory managers.

Conventional Memory

AMIsetup stays in real mode and reads the BIOS from F000:0000 (or

E000:0000). Problems are to be expected if a memory manager or

shadow RAM is enabled.

Image File

AMIsetup reads the BIOS from a file, which can be created by e.g.

`Tools|Create BIOS image' If the image is OK, this mode shouldn't

cause any trouble.

Password

You can define a password under `System|Options', which will be

needed at program start. Password protection reduces misuse of the

AMIsetup.

Changes of the password are written into the file AMISETUP.EXE

that must not be write protected for this operation.

3.1.2 Video mode

Toggles the display mode between 25 lines and 43/50 lines and

requires an EGA/VGA graphics adaptor. AMIsetup automatically stores

the last used video mode.

3.1.3 Reboot

Executes a cold boot. The keyboard controller is programmed to

trigger a hardware reset.

The `Reboot' function can be turned off using `System|Options'.

CAUTION! If you use a hard disk cache or hard disk doubler, this

function could possibly cause data loss.

AMIsetup first tries to flush the SmartDrive cache (if it is

installed, you can disable this feature using the command line

argument `-NOSM'), waits 2 seconds and then resets the computer.

Under OS/2, only the program is closed.

3.1.4 Exit

Exits the program.

3.2 Diagnostic menu

�������������������

Diagnostic

�����������������������������Ŀ

� BIOS version                � 3.2.1

�����������������������������Ĵ

� CMOS RAM checksums          � 3.2.2

� CMOS RAM information        � 3.2.3

�����������������������������Ĵ

� Keyboard controller         � 3.2.4

� Co-processor status         � 3.2.5

� DMA clock speed             � 3.2.6

� Compare DOS/real time clock � 3.2.7

�������������������������������

3.2.1 BIOS version

Verifies whether the BIOS and AMIsetup are compatible. This check

is executed automatically whenever you start Setup.

If this test fails, ensure the BIOS source is set to `Protected

Mode' under `System|Options'.

See the trouble shooting section (appendix C) for more information

about compatible BIOS versions.

3.2.2 CMOS RAM checksums

Verifies the CMOS RAM checksums. There are two areas - the first is

in all AT computers, the second only in PCs with an AMI BIOS. If one

of these two tests fails, the program declines to run Setup.

If you have written your Setup to a file before this error occurred,

you can correct the problem through `Backup|Restore CMOS RAM'.

AMIsetup saves your configuration automatically

- when you first use the program into the file SAVECMOS.SAV

- whenever you invoke the program into the file CMOSBKUP.SAV.

See the trouble shooting section (appendix C) for information on

possible errors.

3.2.3 CMOS RAM information

Shows the date of the RTC, the alarm time and information about

errors at system boot.

����� Information about CMOS RAM contents ����ͻ

�                                              �

�  Date �������������������Ŀ                  �

�  � 09/16/1995             �                  �

�  ��������������������������                  �

�                                              �

�  Alarm ������������������Ŀ                  �

�  � 00:00:00           off �       OK    �    �

�  ��������������������������    ����������    �

�                                              �

�  Status �����������������Ŀ     Print   �    �

�  � Battery:             � �    ����������    �

�  � Power:               � �                  �

�  � Checksum:            � �      Help   �    �

�  � Configuration:       � �    ����������    �

�  � Memory size:         � �                  �

�  � Hard disk C:         � �                  �

�  � Time:                � �                  �

�  ��������������������������                  �

����������������������������������������������ͼ

Date: The current system date of the real time clock (RTC).

Alarm: The RTC supports an `alarm clock' mode. You need an external

program (TSR) that processes the RTC message and then activates.

Status: Shows possible errors from the last system boot. A `�' means

`no error found'.

- Battery: CMOS battery is discharged and CMOS contents were invalid

- Power: RTC power supply failed

- Checksum: CMOS checksums were invalid

- Configuration: Self test results and CMOS configuration were

different

- Memory size: Memory size determined by self test and memory size

stored in CMOS were different

- Hard disk C: Hard disk C: (if available) couldn't be initialized and

couldn't be booted from

- Time: Time was invalid

Errors often disappear after a reboot (e.g. Checksum).

3.2.4 Keyboard controller

Tries to detect vendor and revision of the keyboard controller.

AMIsetup knows chips from American Megatrends, Inc. and Award, Inc.

All other vendors are shown as `unknown'.

This function is disabled under OS/2.

3.2.5 Co-processor status

This function performs a coprocessor hardware check and compares the

result with the BIOS coprocessor presence bit, which is set during

system boot.

If both results correspond (i.e. `Coprocessor present' and `Status

bit set' or `No coprocessor present' and `Status bit not set'),

everything is OK.

If both results do not correspond (`*ERROR*' is displayed in

addition), the BIOS most probably doesn't sense the coprocessor.

If you have problems with applications which support a math

coprocessor and the above error is shown, you should first look into

`Setup|Extended Setup� and see if you can enable `Numeric Coprocessor'

or `Numeric Coprocessor Test'.

If no such setting is available, you can create a small `SETNPU'

program and add it to your AUTOEXEC.BAT.

Please copy all characters exactly as shown:

C:\> DEBUG SETNPU.COM

The specified file name expression was not found.

-a

....:0100 mov ax,40

....:0103 mov ds,ax

....:0105 or byte ptr [10],2

....:010A mov ax,4c00

....:010D int 21

....:010F                     [Press Enter]

-rcx

CX 0000

:0f

-w

Writing 000F bytes

-q

C:\>

3.2.6 DMA clock speed

Tries to determine the current DMA clock speed (this operation is

very hardware dependent and it is possible it won't work - `Error'

is displayed then).

The measured clock speed is displayed in

green  (up to 4.25 MHz): Everything OK

yellow (up to 5.10 MHz): Caution, possibly decrease DMA clock

speed

red    (from  5.10 MHz): The clock is too fast and can cause

damage!

Conditioned by `DMA waits' it is possible that too low a clock is

shown (especially on fast i486 systems). I.e.: If AMIsetup detects a

`red' DMA clock, then it is truly time to decrease it - the DMA chip

can be destroyed by overclocking!

On many systems, DMA clock is coupled with bus clock (e.g. ETEQ,

OPTI, SIS, UMC).

You can compute the bus clock using the following formula:

Bus Clock = 2 * DMA Clock

Depending on the DMA waitstates, the factor can be 2.75 (1 Waitstate)

or 3 (2 Waitstates) instead of 2.

It is neither possible to measure the bus clock nor the number of

DMA waitstates directly.

This function is disabled under OS/2.

3.2.7 Compare DOS/real time clock

Compares the current DOS time and the RTC time and is used for

reference purposes only. If the difference is more than about 2

seconds, you should check hard and software.

3.3 Backup menu

���������������

Backup

�����������������������Ŀ

� Save CMOS RAM...      � 3.3.1

� Restore CMOS RAM...   � 3.3.2

�����������������������Ĵ

� Save EISA NVRAM...    � 3.3.3

� Restore EISA NVRAM... � 3.3.3

�������������������������

3.3.1 Save CMOS RAM...

Saves the CMOS RAM to a file. You can change the default directory

under `System|Options'.

3.3.2 Restore CMOS RAM...

Reads an image file back to CMOS RAM and restores the saved state.

Time and date are not changed.

3.3.3 Save and Restore EISA NVRAM...

On some EISA computers, you can also save and restore the EISA NVRAM

(parts of the configuration data are stored in the EISA NVRAM on

these machines).

See appendix H for more information.

3.4 Setup

���������

������������������������������������ Setup ����������������������������

�05/05/91(C)1990 American Megatrends Inc., All Rights Reserved

�

�

�

�

�                      �������������������������������Ŀ

�                      �        Standard Setup         �

�                      �    Extended/Chipset Setup     �

�                      �        Change Register        �

�                      �           Password            �

�                      �    Load ROM Setup Defaults    �      WARNING:

�                      �  Load ROM Power-On Defaults   �      Improper

�                      �������������������������������Ĵ      Setup may

�                      �      Print Current Setup      �      severe pr

�                      �    Print Reference Manual     �

�                      �        Analyze BIOS ID        �

�                      �������������������������������Ĵ

�                      �     Save Changes and Quit     �

�                      �      Exit without Saving      �

�                      ���������������������������������

� 30-0201-ABCDEF-00101111-050591-OPWB

�����������������������������������������������������������������������

Please read the warnings at the beginning of this document.

3.4.1 Standard setup

��������������������

You can change the following settings (to change date and time,

please use the DOS commands DATE and TIME).

��[�]��������������������� Standard setup ������������������������ͻ

�                                                                  �

� Base memory:       640K                                          �

� Extended memory: 16384K           Auto detect hard disks �       �

�                                   ������������������������       �

� Drive A         Drive B           HDD 0 �         HDD 1 �        �

�                                   �������         �������        �

�  ( ) None        ( ) None          Type:    47    NONE / SCSI    �

�  ( )  360K 5�"   ( )  360K 5�"     Cyl:   1024                   �

�  ( )  1.2M 5�"   (x)  1.2M 5�"     Heads:   64                   �

�  ( )  720K 3�"   ( )  720K 3�"     S/Trk:   26                   �

�  (x) 1.44M 3�"   ( ) 1.44M 3�"     Size:  832M                   �

�  ( ) 2.88M 3�"   ( ) 2.88M 3�"                                   �

�                                                                  �

� Display adapter      Options                                     �

�  ( ) Not installed    [X] Keyboard installed                     �

�  ( ) Color 40x25                                                 �

�  ( ) Color 80x25                                                 �

�  ( ) Monochrome                                                  �

�  (x) VGA/PGA/EGA                  OK   �    Cancel �     Help  � �

�                                 ��������    ��������    �������� �

������������������������������������������������������������������ͼ

At top, the memory size as determined by the BIOS is displayed.

Drive A

Drive B

Allows you to change the type of installed floppy disk drives.

2.88 MB disk drives:

Using AMIsetup, you can configure the drive type `2.88 MB' on

any computer. To run a ED drive, you need a diskette controller

capable of the 1 Mbps data rate (e.g. AHA1542CF).

If your BIOS does not support 2.88 MB drives, AMIsetup displays

a warning message (BIOS versions earlier than 06/12/91).

Lack of BIOS support can cause the following problems:

- Problems booting from diskette (especially 720 KB diskettes)

- The built-in ROM `Standard setup' displays a line containing

garbage characters (this does not adversely affect other setup

functions).

- You'll need software support for your floppy drive. OS/2

automatically detects 2.88 MB drives. DOS 5.0 or later requires

a device `DRIVER.SYS' (please consult your DOS manual). The

author is currently developing a flexible BIOS enhancement which

must be installed in CONFIG.SYS, supporting 360 KB to 2.88 MB

floppy drives on any port, IRQ or DMA channel.

- Dependent on your hardware, it might be better instead to tell

the BIOS the drive was 1.44 MB.

You can disable the warning message (displayed if your BIOS does

not support 2.88 MB drives) using the command line parameter

`-288'.

Display adapter

With this you set the type of the installed graphics adapter.

( ) Not installed

( ) Color 40x25

( ) Color 80x25   = CGA

( ) Monochrome    = MDA, Hercules

(x) VGA/PGA/EGA

On older main boards, you have to change a jumper in addition to

this setting.

Options - Keyboard installed

If the keyboard should be checked at system boot, check this box.

Should the keyboard not be checked or if there is no keyboard

installed (e.g. LAN server), leave it empty.

Auto detect hard disks

You don't need to enter the parameters of your hard disks manually

if you have IDE type drives installed.

AMIsetup can detect the correct cylinder/head/sector values

automatically (non-OS/2 systems only).

��[�]����������������� Confirm ���������������������ͻ

�                                                    �

�               Install the following                �

�              hard drives into setup?               �

�                                                    �

�  HDD 0: Conner Peripherals 170MB, CP30174E         �

�  HDD 1: Not installed / SCSI                       �

�                                                    �

�                                                    �

�           Yes   �      No    �     Help   �        �

�         ���������    ���������    ���������        �

����������������������������������������������������ͼ

If no hard disks are found, it's most likely you have SCSI type

drives (or old MFM or RLL drives).

In the case of MFM/RLL type drives, you must enter the disk

parameters manually; this also holds true for some SCSI

controllers.

HDD 0

HDD 1

With this you can set the type of installed MFM, RLL or IDE

hard drives. Using a SCSI controller, you mustn't define a hard disk

here - please read your controller and hard disk manual to prevent

momentous wrong settings!

The current parameters are displayed below the push-button - type

(number) of the disk, cylinders, heads, sectors per track and the

calculated size.

Note: The disk size is displayed in real Megabytes, 2^20 bytes.

Many disk vendors calculate in 10^6 bytes (this number is larger!).

If you push the button, a list of predefined disk types appears:

��[�]��������� Hard disk 0 parameters ������������ͻ

� Type� Cyls. �Heads� Precomp� LZone �S/Trk�  Size �

��������������������������������������������������ĺ

�  32 �  1024 �  15 �   NONE �  1024 �  17 �  128M >

�  33 �  1024 �   5 �   1024 �  1024 �  17 �   43M �

�  34 �   816 �  15 �   NONE �   816 �  32 �  191M �

�  35 �  1024 �   9 �   NONE �  1024 �  17 �   77M �

�  36 �  1024 �   8 �    512 �  1024 �  17 �   68M �

�  37 �   615 �   8 �    128 �   615 �  17 �   41M �

�  38 �   745 �   4 �    512 �   745 �  28 �   41M �

�  39 �   987 �   7 �    987 �   987 �  17 �   57M �

�  40 �   820 �   6 �    820 �   820 �  17 �   41M �

�  41 �   977 �   5 �    977 �   977 �  17 �   41M �

�  42 �   981 �   5 �    981 �   981 �  17 �   41M �

�  43 �   755 �  16 �   NONE �   755 �  17 �  100M �

�  44 �   887 �  13 �   NONE �   887 �  34 �  191M �

�  45 �   968 �  10 �   NONE �   968 �  34 �  161M �

�  46 �   751 �   8 �      0 �   751 �  17 �   50M �

�� 47 � USER DEFINED�        �       �     �      �<

��������������������������������������������������ͼ

The columns contain type (number) of the disk, number of cylinders,

heads, the precompensation cylinder (see glossary), landing zone,

number of sectors per track and the calculated size (rounded) in

megabytes. Please compare these items with the data sheet of your

hard disk.

It's likely you CANNOT find the specifications of the hard disk to

be installed in this list. In this case, select type 47 (USER

DEFINED).

The following dialogue appears:

��[�]�������  Change type 47 parameters  �����������ͻ

�                                                    �

�                                                    �

�     615 �Cylinders                                 �

�                                                    �

�       4  Heads                                     �

�                                                    �

�     300  Precompensation (65535 for NONE)          �

�                                     [  Save   ]    �

�     615  Landing zone                              �

�                                     [ Cancel  ]    �

�      17  Sectors/track                             �

�                                     [  Help   ]    �

�                                                    �

����������������������������������������������������ͼ

Here you can set the parameters yourself. Note that the value of

precompensation has a special meaning: 65535 means DISABLED for the

whole hard disk. 0 means ENABLED for the whole disk.

Modern drives ignore precompensation and landing zone. For reasons

of software compatibility, set precompensation to `NONE' and landing

zone to the highest available cylinder.

3.4.2 Extended/chipset setup

����������������������������

A list of options and their current settings is displayed:

��[�]����������������������� Extended setup ���������������������������

�            Option            �Register�Bits � Current setting

�����������������������������������������������������������������������

��  Typematic Rate Programming � 11  10000000 � 10000000  Enabled     �

�   Typematic Rate Delay (msec)� 11  01100000 � 00000000  250

�   Typematic Rate (Chars/Sec) � 11  00011111 � 00000011  21.8

� � Mouse Support Option       � 13  10000000 � 10000000   <1>

�   Above 1 MB Memory Test     � 13  01000000 � 00000000  Disabled

�   Memory Test Tick Sound     � 13  00100000 � 00000000  Disabled

�   Memory Parity Error Check  � 13  00010000 � 00010000  Enabled

�   Hit <ESC> Message Display  � 13  00001000 � 00000000  Disabled

�   Hard Disk Type 47 Data Area� 13  00000100 � 00000000  0:300

�   Wait For <F1> If Any Error � 13  00000010 � 00000010  Enabled

�   System Boot Up Num Lock    � 13  00000001 � 00000001  On

� � Numeric Processor          � 14  00000010 � 00000010  Present

�   Weitek Processor           � 2D  10000000 � 00000000  Absent

�   Floppy Drive Seek At Boot  � 2D  01000000 � 00000000  Disabled

�   System Boot Up Sequence    � 2D  00100000 � 00000000  C:, A:

�   System Boot Up Speed       � 50  00000100 � 00000000  High

�   Cache Memory               � 2D  00001000 � 00001000  Enabled

� � Internal Cache Memory      � 2D  00000100 � 00000100   <1>

�����������������������������������������������������������������������

The four columns have the following meaning:

Option

The name of the setting to be changed, peeked from the BIOS.

The signs `�', `�' and `x' in the first column have a special

meaning:

`�' shows that the option is hidden in the built-in BIOS setup.

The `�' options work correctly in most cases.

`�' shows that the option is hidden in the built-in BIOS setup and

that AMIsetup cannot find a textual description of the possible

settings. Often `�' options don't work.

If you change settings of options marked in this way, be especially

careful. You can disable the display of these options selectively

under `System|Options'.

`x' shows that the BIOS has currently deactivated the option (e.g.

because the option is only valid for a 486 processor, but there is a

386 processor installed or because the option can only be activated

in conjunction with some other option).

By the way, blame the BIOS manufacturer for reversed letters in this

column (however, AMIsetup has a built-in spell checker since version

2.99).

Register

Hexadecimal number of the CMOS register that stores the setting.

This information may help experienced users to change settings

manually using Change Register. If a `+' sign is displayed after the

register number, there might be other registers affected.

Bits

Bit mask for the setting. Only Bits marked with a `1' are used for

the specified setting (see also Register).

Current setting

Bit mask of the setting stored in the program copy of CMOS RAM and

the corresponding text in clear. If plain language is not available,

a decimal value in acute brackets is shown. Example: <2> denotes the

third possible setting, since <0> is valid, too.

Now you can select an option with the mouse or the enter key, as

appropriate. A new window is displayed (see below).

Instead of pressing the enter key, <Ctrl><RightArrow>, <Ctrl><Enter> or

<Ctrl><LeftArrow> increase or decrease the current setting respectively

by one (for the hurried user).

List:                                       Radio buttons:

��[�]�������� Typematic Rate (Chars/Sec)    ��[�]���� Typematic Rat

�                                           �

�   �Options                 Bits           �   �Options

�     30.0             <     00000011       �    �(x) Disabled�

�     26.7             �    �00011111�      �     ( ) Enabled

�     24.0             �                    �

�    �21.8            ��              [     �

�     20.0             �                    �

�     18.5             �              [     �

�     17.1             �                    �

�     16.0             �              [     �

�     15.0             >                    �

�                                     [     �

�                                           �

����������������������������������������    �����������������������

Marked below `Options' or `Setting' is the current setting. To change

the setting, scroll the list or press the appropriate button.

The `Bits' field shows you which bit mask corresponds to the selected

setting. Listed below the bit field are once again the bits that can be

affected by the option.

`Save' copies the changed setting into the program copy of CMOS RAM

(use Save Changes and Quit to make the change permanent), `Cancel'

cancels the operation.

Caution: Some settings are `Reserved' or `RESERVED'. Do consider

whether you really want to set an option to `Reserved'. For safety

reasons, these changes have to be confirmed.

`BIOS help': Sometimes, the BIOS ROM has an own (English) help text to

the respective option. You can view this text using `BIOS help', e.g.:

��[�]��������� BIOS help �������������ͻ

�The base address changes in           �

�steps of 16k and 512k                 �

��������������������������������������ͼ

`Reference': Registered users can look up explanations to the setup

options (only one in five explanations will be shown to unregistered

users).

Options that are controlled entirely by the BIOS (these usually change

more than eight bits and are marked with a `+') display a window

containing the buttons `� < > �'; you can also use (<Ctrl>)<RightArrow>

and (<Ctrl>)<LeftArrow>. The buttons `� �' are intended for fast

leafing and don't work on all machines. On some machines, a window

appears after pressing `� < > �'. Here you can enter data - this input

window is controlled by the BIOS, so mouse input and function keys are

disabled.

Setting controlled by BIOS:

��[�]�������� Non-Cacheable Block-0 Base �����������ͻ

�                                                    �

�                                                    �

�                                                    �

�                                      [BIOS help]   �

�   Setting                                          �

�   _0 KB_______________________       [Reference]   �

�                                                    �

�    [ � ]  [ < ]  [ > ]  [ � ]        [  Save   ]   �

�                                                    �

�                                      [ Cancel  ]   �

�                                                    �

�                                      [  Help   ]   �

�                                                    �

����������������������������������������������������ͼ

Note that some options affect each other!

3.4.3 Change register

���������������������

You may not be able to set the CMOS the way you want from the

`Extended/Chipset Setup' menu. In this case, you can make the change

manually.

Caution: Use this function only if you are absolutely, positively sure

about what you're doing!

First, you have to enter the register number in hexadecimal and then

the new value.

3.4.4 Password

��������������

This function enables you to display the current CMOS password and to

change it.

This password can be called for at system boot or when entering the

ROM setup.

��[�]���������� Password �������������ͻ

�                                      �

� Current password is `AMISET'.        �

�                                      �

�                                      �

�                                      �

�  [Change]    [Cancel]    [ Help ]    �

�                                      �

��������������������������������������ͼ

��[�]������ Change password ����������ͻ

�                                      �

�  New password: FIDO__                �

�                                      �

�                                      �

�                                      �

�  [  OK  ]    [Cancel]    [ Help ]    �

�                                      �

��������������������������������������ͼ

Caution! If you use a foreign language keyboard (i.e. non-USA),

note that no foreign keyboard support is loaded at boot time.

Example: On German keyboards, `Z' and `Y' are reversed.

Often you can enable the US-American keyboard by pressing

<Ctrl><Alt><F1> and then test which keys are which. Use <Ctrl><Alt><F2>

to re-enable the foreign keyboard.

On some machines the password function has to be activated by the

`Password' option in `Extended/Chipset Setup'.

CAUTION! If you set the `Password' option to `Always', you cannot

boot your machine if you forget the password!

Hint: Try the setting `Setup' first.

3.4.5 Load ROM Setup Defaults, Load ROM Power-On Defaults

���������������������������������������������������������

There are two tables of predefined CMOS settings in the ROM. One of

those, the `power-on defaults' is used to eliminate all possible

hardware problems; thus the processor is switched to low speed, the

cache is turned off, etc. Using `power-on defaults', the machine should

boot in any case.

Using this function, you can load a predefined configuration or simply

view it for reference purposes, since all settings will be written to

the program copy of CMOS RAM first and only be saved if you use `Save

Changes and Quit'.

3.4.6 Print current setup

�������������������������

With this function, you can print the current settings of the program

copy of CMOS RAM.

�  Format                             �

�   (x) Standard (current/possible)   �

�   ( ) Alternate (current bin&hex)   �

First the settings of Standard setup will be printed; then, a list:

Standard: The list is formatted as follows:

Option             � Current  � Possible settings

����������������������������������������������������������������

Typematic Rate Programming   � Enabled  � Disabled

Typematic Rate Delay (msec)  � 250      � 500 750 1000

Typematic Rate (Chars/Sec)   � 24.0     � 30.0 26.7 21.8 [...]

� Mouse Support Option         � <1>      �

� Above 1 MB Memory Test       � Disabled � Enabled

���

`Margin' (61-254) is the maximum width of the list. `Possible settings'

will be truncated to `[...]' if exceeding the maximum width.

Alternate: The list looks as the list displayed in Extended/Chipset

Setup. `Margin' is ignored.

Option             �Register�Bits � Current setting

������������������������������������������������������������������

Typematic Rate Programming   � 13  10000000 � 10000000  Enabled

Typematic Rate Delay (msec)  � 13  01100000 � 00000000  250

Typematic Rate (Chars/Sec)   � 13  00011100 � 00010000  15

� Mouse Support Option         � 11  10000000 � 10000000  Enabled

���

Hint: If you disable `�' and `�' under `System|Options', you can create

a list of those settings that can be changed from the ROM setup. For

example, you could distribute this list to your customers as a

reference.

3.4.7 Print Reference Manual

����������������������������

This function prints a personal BIOS manual (the unregistered

version prints only one in five explanations).

Please read the license restrictions!

3.4.8 Analyze BIOS ID

���������������������

This function explains the meaning of the BIOS ID string displayed

in the lower left corner.

Note: The information determined by AMIsetup is not always 100 per cent

correct, since not all manufacturers adhere to the standard (this is

especially true for `Required processor').

The ID string displayed at boot time shows the keyboard controller

revision level in addition to the information displayed by

AMIsetup (see also `Diagnostic|Keyboard controller')

Example: 30-0201-ABCDEF-00101111-050591-OPWB-KF, where F is the

revision level.

���������������� Analyze BIOS ID string ��������������ͻ

�                                                      �

�  30-0201-ABCDEF-00101111-050591-OPWB                 �

�  23H-0-0000-00-00-0000-00-00-000                     �

�  23H-1-0000-00-00-0000-00-00-00-2                    �

�                                                      �

�  Required processor:   386                OK    �    �

�  BIOS size:            64 KB           ����������    �

�  Version:              2.01                          �

�  Reference number:     ABCDEF           Print   �    �

�  BIOS date:            05/05/91        ����������    �

�  Chipset ID:           OPWB                          �

�  Clock switching pin:  23                Help   �    �

�  Cache control pin:    23              ����������    �

�  BIOS modified flag:   02                            �

�                                                      �

������������������������������������������������������ͼ

Use `Print' to print the information or to write it into a file.

3.4.9 Save Changes and Quit

���������������������������

Not until you use this function will changes be made permanent. `Save

Changes and Quit' writes all CMOS settings from the program copy of

CMOS RAM into the real CMOS RAM and exits setup.

If `System|Options', `Enable reboot' is enabled, you will be asked

whether you  want to reboot your computer (cf. `Reboot').

3.4.10 Exit without Saving

��������������������������

Quits setup without changing the CMOS RAM.

3.5 Tools menu

��������������

Tools

����������������������������Ŀ

� Create BIOS image...       � 3.5.1

����������������������������Ĵ

� Print CMOS RAM contents... � 3.5.2

� Clear CMOS RAM             � 3.5.3

������������������������������

3.5.1 Create BIOS image...

Using this function, you can create a `BIOS image' file. This file is

necessary mainly for diagnostic purposes.

To create a `BIOS image' file, do the following:

1. Select and execute `Tools|Create BIOS image' and enter a file name

(e.g. `ETEQ.ROM' and a description (e.g. `ETEQ-386 baseboard').

2. Select `Protected Mode' as BIOS source.

3.5.2 Print CMOS RAM contents...

Prints the CMOS RAM contents and is used for diagnostic and reference

purposes only. To get a readable, plain English print, use

`Setup|Print current setup'.

Data is written in the following format:

Reg � Current value

�������������������

00 � 13  00010011

01 � 23  00100011

...    ...

7E � 00  00000000

7F � 00  00000000

Opposite to the register number (hexadecimal), the current value is

printed in hexadecimal and binary format.

3.5.3 Clear CMOS RAM

Caution: This function erases the complete CMOS RAM contents,

including checksums, date/time, hard disk parameters, etc. Please

make a paper backup of your configuration before clearing the CMOS

RAM!

3.6 Help menu

�������������

Help

������������������������������Ŀ

� About AMIsetup...  Shift-F10 � 3.6.1

������������������������������Ĵ

� Contents                     � 3.6.2

� View AMISETUP.DOC            � 3.6.3

� Help about registering...    � 3.6.4

� Print order form...          � 3.6.5

������������������������������Ĵ

� On-line reference            � 3.6.6

��������������������������������

3.6.1 About AMIsetup

Shows a dialogue box containing program revision and registration

number (or `Unregistered Shareware').

If you haven't yet paid for AMIsetup and a registration number

appears anyway, you've got an (illegal) pirated copy. Please contact

the author.

3.6.2 Contents

Gives an overview about on-line help.

3.6.3 View AMISETUP.DOC

You can view AMISETUP.DOC without leaving AMIsetup. It is necessary

that AMISETUP.DOC be installed in the same directory as AMISETUP.EXE.

3.6.4 Help about registering

See Appendix A.

3.6.5 Print order form

To order AMIsetup, you can print an order form to the printer or

to a file.

3.6.6 On-line reference

AMIsetup contains explanations of approx. 200 of the most widespread

setup options (unregistered users will see only one in five

explanations).

Select `On-line reference', and the index is displayed.

Next, you can mark one of the bold terms using <Tab> or the mouse and

get a closer explanation. The cursor keys are for scrolling.

4 Key shortcuts

���������������

<F1>         Help

<Alt><F3>    Close active window

<F5>         Zoom active window

<Ctrl><F5>   Move active window (Cursor keys)

Change window size (<Shift>+Cursor keys)

<F9>         Start Setup

<F10>        Activate main menu

<Shift><F10> Show program information

<Alt><X>     Exit Setup or program

or <Alt><F4>

NB:  <#>            = <Shift>

<Return>, <#�> = <Enter>

<#>            = <Tab>

A P P E N D I X

���������������

A  Registration

���������������

Thank your for trying the shareware version of AMIsetup. The shareware

version is lacking some features of the registered version.

You may test the shareware version thoroughly for 30 days. You may

copy it and share it with your friends.

The author makes no money out of the shareware version - if you paid

for it, the payment only covers the shareware vendor's cost of copying

and shipping the software.

If you'd like to purchase the latest registered version, which includes

the complete feature set and does not display this message, please

print the order form using `Help|Print order form'.

Only the registered version entitles you to use the program for more

than 30 days.

If you do not like the program contrary to expectations, just delete it.

But maybe you'd like to suggest a future improvement first, so that the

author will be able to fit the program to your needs.

See the registration form for the registration fee valid for your

country (DM 40.- to DM 55.- depending on the country).

�������������������������������������������������������������������Ŀ

� Please use the form `REGISTER.FRM' included with this program.    �

� You can print it using `Help|Print order form' directly from      �

� AMIsetup.                                                         �

���������������������������������������������������������������������

Japanese users: Please print the file REGISTER.JPN instead.

Commercial users in the European Union ONLY: Please provide your

VAT registration number and save 15% VAT!

Send DM Eurocheques (other cheques: add DM 10.-), cash or postal

money order to the following address:

How to Contact the Author

�������������������������

Please address all correspondence to:

Robert Muchsel

Hegaublick 2

D-78465 Konstanz

Germany

Fax: +49-7533-3151

Electronic Mail (this is a lifetime address):

Internet:   muchsel@acm.org

CompuServe: >INTERNET:muchsel@acm.org

Upgrades

��������

If the user has a modem, upgrades are free. Other registered users

only have to pay the fee of their shareware vendor.

Upgrades will not be free if

- there are too many illegal pirated copies

- there are major enhancements

Read README.!!! on your registration diskette for more information.

The Author

����������

...studies Computer Science at the ETH (Swiss Federal Institute of

Technology) in Zurich/Switzerland (this is the explanation of the

`Additional contributions' field on the order form).

B  AMIsetup and OS/2, Windows, Windows NT

�����������������������������������������

OS/2 Versions 2.1, 2.11, 3

��������������������������

The program will run in a DOS session of OS/2. Some closely hardware

related functions will be disabled to preserve system integrity.

OS/2 Version 2.0

����������������

The program will not run in a DOS session of OS/2 2.0, not even in a

DOS image (VDM). Write accesses to the CMOS RAM are redirected to a

buffer by OS/2, which is discarded when exiting the virtual machine

(source: `Redbooks, Vol. 2: DOS and Windows-Environment, Virtual CMOS

Device Driver'; IBM Document Number GG24-3731-00).

OS/2 in General

���������������

An OS/2 version of AMIsetup is impossible, since AMIsetup directly

executes code of the real mode BIOS.

If you create a DOS boot disk and copy AMIsetup as well as a disk

editor there, then you've got a sort of `emergency disk'.

AMI BIOS and OS/2

�����������������

There are NO general problems with OS/2 and the BIOS versions supported

by AMIsetup. OS/2 runs with BIOS versions PRIOR to 05/05/91 and with

keyboard controller revision -K8.

However, problems might arise e.g. when using faulty memory chips, too

low refresh rates, wrongly installed cache memory, faulty EIDE chips

(on PCI boards) or wrong bus clock. Because of the protected mode

architecture, OS/2 is much more sensitive to errors than DOS software,

which often ignores or doesn't even notice them.

Windows

�������

Running AMIsetup under Windows is not recommended.

The icon file, AMISETUP.ICO, contains an OS/2 icon which cannot

be displayed under Windows.

Windows NT

����������

The Windows NT DOS emulator is not compatible with AMIsetup.

C  Trouble shooting

�������������������

1. Is AMIsetup compatible with your BIOS?

For technical reasons, AMIsetup only works with two generations of

the AMI BIOS, the so-called `Hi-Flex BIOS' and `WinBIOS'. Both versions

are manufactured by `American Megatrends, Inc.' (`AMI'). This you can

see when powering on your computer.

Currently, AMIsetup does not support the AMI-BIOS on later Intel

baseboards (Plato, Endeavour, Premiere, Batman, etc.).

Example of a power-on screen on HighFlex or WinBIOS:

���������������������������������������������������������������Ŀ

� 05/05/91(C)1990 American Megatrends Inc., All Rights Reserved �

�                                                               �

� (C)1990 American Megatrends Inc.,                             �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

� 30-0101-ABCDEF-00011001-050591-FORX                           �

�����������������������������������������������������������������

The HighFlexBIOS has three levels of setup (`Standard', `Advanced' and

`Advanced Chipset'), `Password' and `Hard Disk Utility'.

��������������������������������������������������������������ͻ

�          BIOS SETUP PROGRAM-AMI BIOS SETUP UTILITIES         �

�    (C) 1990 American Megatrends Inc., All Rights Reserved    �

��������������������������������������������������������������͹

�                                                              �

�                     STANDARD CMOS SETUP                      �

�                     ADVANCED CMOS SETUP                      �

�                   ADVANCED CHIPSET SETUP                     �

�            AUTO CONFIGURATION WITH BIOS DEFAULTS             �

�          AUTO CONFIGURATION WITH POWER-ON DEFAULTS           �

�                       CHANGE PASSWORD                        �

�                      HARD DISK UTILITY                       �

�                    WRITE TO CMOS AND EXIT                    �

�                DO NOT WRITE TO CMOS AND EXIT                 �

�                                                              �

��������������������������������������������������������������͹

�                                                              �

�  Standard CMOS Setup for Changing Time, Date, Hard Disk, etc.�

�                                                              �

�����͵ ESC:Exit |-|-:Sel F2/F3:Color F10:Save & Exit ��������ͼ

Older, custom-made BIOS versions are NOT compatible with AMIsetup, even

though they are from AMI. These have a so-called `XCMOS Setup' (and they

are black & white).

The setup built into the WinBIOS has Windows-like icons and should be

easily recognized.

2. Frequent errors with compatible BIOS versions

The BIOS source is not correctly set. Verify that the BIOS source under

`System|Options' is set to `Protected Mode'.

3. Stopgap: -O

As a last resort you can start the program with the command-line option

`-O' (CAPITAL letters). This option disables all error messages; faulty

operation or a locked-up computer may result, though.

4. Additional trouble shooting options

If the program hangs when you start the `Extended setup', you may want

to try the command line argument `-N'.

D  Glossary

�����������

Some frequently used abbreviations and terms (explanations for setup

options only on-line):

AMI

American Megatrends Inc., a BIOS manufacturer

BIOS

Basic routines contained in a nonvolatile memory (ROM) of the PC

which e.g. drive the hard disk, but are also needed for system boot

and system setup.

BIOS Checksum

The AMI BIOS calculates a 16 bit checksum (addition w/ overflow) over

the BIOS ROM. If this checksum is 0, it is assumed that the BIOS

is OK.

CMOS RAM

This chip can be read from and written to; unlike normal memory the

contents are buffered by a battery or an accumulator after turning

off the PC. CMOS RAM contains data like type of installed floppy

disk drives, the time, etc.

CMOS RAM Checksum

A checksum is calculated for parts of the CMOS and stored in CMOS.

If checksum and CMOS RAM contents differ, either the battery is

defect or the CMOS RAM contents are invalid. In any case you must

reenter the data, possibly replace the battery or accumulator,

too.

EISA

See appendix H.

Extended Character Set, Graphic characters

Characters like ��� or ��. Some printers cannot print these.

NVRAM

See appendix H.

Precompensation

Precompensation is used with some hard disks to compensate for the

higher recording density in the inner tracks during write.

Normally, precompensation and reduced write current begin

approximately at half the cylinder count. The precompensation

logic shifts certain bit patterns by +/- 12-15ns.

Precompensation only affects writing, not reading of data.

Many hard drives work perfectly with or without precompensation,

newer hard disks ignore the setting (you should turn it off then).

ROM

Read Only Memory - memory that can only be read from and not

written to.

E  If the machine won't boot...

�������������������������������

If the setup has been altered so that the PC doesn't even display

`Press <DEL> for Setup', some models have the following possibilities:

1. On newer models, you can press <Ins> while powering on. All

settings will be reset to standard values.

2. On many machines there is a jumper on the main board which you can

use to erase CMOS RAM. Sometimes this takes several hours.

3. Often there is the possibility of connecting an external battery

pack. To install it, you have to remove a jumper residing on a

4 pinned connector. This jumper does the same as (2).

4. Only when encountering password errors and only with BIOS from

12/12/91 on: Pin 32 of the keyboard controller can be grounded

to erase the password.

Please note that after these steps all configuration data must be

reentered. Make certain that you set the correct hard disk parameters!

If you are not a hardware expert, you should contact your dealer first

before destroying your hardware by wrong handling!

F  Switching to the English user interface

������������������������������������������

If you have installed the German version and want to switch,

do the following:

Disable `German language' or `Deutsche Oberfl�che' under

`System|Options'. Select OK, exit and restart AMIsetup.

G  Soundblaster

���������������

To make the monotonous task of setting up computers a little more fun,

AMIsetup supports the Soundblaster audio card (by Creative Labs).

Soundblaster support must be explicitly enabled (AMIsetup is, after

all, a serious program).

- Turn on `Soundblaster' under `System|Options' (default: OFF)

- The environment variable `BLASTER' must be set correctly, e.g.: SET

BLASTER=A220 I5 D1

The BLASTER variable has the following meaning:

A220 : Port address of Soundblaster card

I5   : IRQ (Interrupt) level

D1   : DMA channel

(all other settings, e.g. T4, will be ignored)

If the variable is NOT set, the default values A220 I7 D1 will take

effect.

Note: The program could crash if a wrong DMA channel is given!

- You must have the VOC files. To save disk space and transfer time,

the unregistered version contains only a small subset of the files.

All versions include:

ERROR.VOC    - Replaces the error beep

In addition, the registered version is shipped with:

CONFIRM.VOC  - Confirmation

COPYRIGH.VOC - About AMIsetup

ERRORBOX.VOC - Error

EXIT.VOC     - Program exit

WARNING.VOC  - Warning

Not included:

HELP.VOC     - Help

AMIsetup has its own, built-in Soundblaster driver. This built-in

driver should work with all Soundblaster and compatible cards with DSP

level 1.05 and higher.

If the driver CTVDSK.DRV is included with your Soundblaster card,

AMIsetup can use it and adapt itself to the special features of your

card (e.g. Stereo sound).

AMIsetup looks for CTVDSK.DRV in the following directories:

1. in the current directory

2. in the directory of AMISETUP.EXE

3. in the subdirectory `DRV\' of the directory specified by the

environment variable `SOUND'

4. in the subdirectory `DRV\' of the directory specified by the

environment variable `CPSPRO4'

5. in the PATH

The built-in driver will only be activated if CTVDSK.DRV cannot be

found.

You can delete single VOC files and the corresponding action will be

`quiet'.

You can replace these files with files from your own sound collection.

AMIsetup can play files any size you want; if disk activity is too

heavy, you can increase the replay buffer which defaults to 4*4 KB.

Set the DOS enviroment variable SBBUF to the desired value.

Example: SET SBBUF=16 -> replay buffer will be set to 4*16 KB.

All sounds are turned off immediately when the corresponding action is

aborted/finished.

If the program hangs when you activate Soundblaster support, you can

force the program to use its built-in driver. Type `SET SOUND=' at

the DOS command prompt.

If you cannot get sound support to work at all, don't be sad -

AMIsetup's real functionality will not be affected by this gadget.

H  EISA

�������

EISA is an `Extended' and faster bus architecture found in better

machines. AMIsetup tells you whether you have a standard ISA, a

PCI or EISA system.

On some EISA computers, parts of the setup data are stored in the

EISA NVRAM (Non Volatile RAM). On these machines, AMIsetup needs to

access the NVRAM. On others, AMIsetup doesn't need to know about EISA

NVRAM and simply ignores it.

Since EISA NVRAM access is chipset dependent, AMIsetup uses EISA.CFG

(see EISA.CFG for explanation of the syntax).

If AMIsetup detects an EISA system and EISA.CFG contains valid

data, `Backup�Save EISA NVRAM...' and `Backup�Restore EISA

NVRAM...' are automatically enabled. Setup registers 40h to 7F are

mapped into the EISA NVRAM.

However, it is possible that AMIsetup doesn't know about YOUR

particular EISA computer and displays `The BIOS register table

contains invalid entries!'. Please mail a copy of your BIOS.DAT,

CMOS.SAV and EISANVR.SAV (if available). EISA.CFG can then be extended

to include your computer.

I   Batch mode

��������������

You can use AMIsetup in batch mode. This is especially useful if you

- want to configure several identical machines (you don't have to use

time consuming ECU, CMOS setup any more!)

- want to document CMOS settings and/or save CMOS/EISA NVRAM

automatically

The batch mode of AMIsetup understands the following commands:

-SAVE:CMOS:FILENAME          Save CMOS RAM into FILENAME.

-SAVE:EISA:FILENAME          Save EISA NVRAM into FILENAME.

-RESTORE:CMOS:FILENAME       Restore CMOS RAM from FILENAME.

-RESTORE:EISA:FILENAME       Restore EISA NVRAM from FILENAME.

-PRINT:DEVICE                Print setup to DEVICE.

-PRINT:ALT:DEVICE            Print setup (alternate style) to

DEVICE.

Examples:

You want to save CMOS & EISA NVRAM in a file:

AMISETUP -SAVE:CMOS:CMOS.BIN

AMISETUP -SAVE:EISA:EISA.BIN

And now configure several more machines with the same settings:

AMISETUP -RESTORE:CMOS:CMOS.BIN

AMISETUP -RESTORE:EISA:EISA.BIN

Finally, print setup to a parallel printer:

AMISETUP -PRINT:LPT1

J   Obtaining new or bug fixed program revisions

������������������������������������������������

There are lots of different BIOS/chipset combinations out there.

If the program doesn't work with your machine, please do the

following:

1. Read the manual, especially appendix C (you did, didn't you?).

2. Use the latest version of the program only. You can get it from

your shareware vendor or, if you have a modem, from the mccs BBS

in Singen/Germany.

mccs BBS Singen/Germany (Sysop Martin Grass, Fido 2:246/8100-8104)

Line 1: Phone +49-7731-65833

FidoNet users: Request the file using the `magic' name AMISETUP.

Others: Please log in as `GAST', password `GAST' and download

AMISETUP directly from the main menu.

You can also get the AMIsetup shareware from the Internet.

SimTel ftp server and its mirrors (e.g. nic.switch.ch) carry the

latest version in the directory msdos/sysutl.

3. ONLY if the latest version doesn't work and you are sure it should

work (appendix C!):

Send a BIOS image and a CMOS RAM image produced by AMIsetup to

the author.

Please follow the instructions in the manual; ensure that NO

memory manager is loaded.

Thank you.

K   Tested BIOS revisions

�������������������������

Among others, the program has been successfully tested on the

following chip set/BIOS combinations (the list was generated

automatically):

Chip set ID- BIOS Date

4X521      - 04/04/93

80486      - 07/07/91

80486      - 12/15/93

80X86      - 06/06/92

ACC2046    - 06/06/92

ACC2046    - 12/12/91

ACER1217   - 07/07/91

ALI1419    - 12/12/91

ALI1429    - 08/08/93

AMI/S50    - 12/12/91

AMI/S61    - 12/12/91

BENG386    - 12/12/91

BENG486    - 12/12/91

CAESAR     - 07/07/91

CTQ386&486 - 12/12/91

CTQ596     - 06/06/92

ET/386H    - 02/02/91

ET/486H    - 05/05/91

FORX       - 05/05/91

FORX       - 07/07/91

ITOPDX     - 05/05/91

MERCISA    - 12/15/93

NxVL       - 11/11/92

O546822    - 06/15/94

O546822    - 07/25/94

OP802G     - 12/15/93

OPEISA     - 12/12/91

OPT4 498   - 06/06/92

OPT495SX   - 11/11/92

OPTI597    - 08/08/93

OPTI802    - 07/25/94

OPTi895    - 12/15/93

OPTIEISA   - 07/07/91

OPWB       - 05/05/91

OPWBSX     - 06/06/91

OVIPER     - 10/10/94

PCI82420   - 01/25/94

PCI82430   - 04/06/94

PCI82430   - 06/16/94

PEAKDM_B   - 05/05/91

PICO168    - 06/06/92

SATURN     - 08/08/93

SATURN_1   - 12/15/93

SHASTA     - 11/11/92

SIS-EISA   - 06/06/92

SIS-EISA   - 12/12/91

SIS3486    - 06/06/92

SIS3486    - 11/11/92

SIS401     - 08/08/93

SIS461     - 08/08/93

SIS471     - 07/25/94

SIS471     - 12/15/93

SIS471G    - 07/25/94

SIS486     - 07/07/91

SIS496AB   - 07/25/94

SIS50X     - 07/25/94

SIS550X    - 07/25/94

SISD       - 07/07/91

SISEISA    - 06/06/92

SUMMIT_C   - 12/12/91

SYM-486    - 05/05/91

SYM486B    - 07/07/91

SYMP       - 03/15/91

SYMP       - 05/05/91

SYMP       - 12/12/91

SYM_386B   - 05/05/91

TIEISA/S   - 12/12/91

TOPCATSX   - 05/05/91

U4800VLX   - 11/11/92

UMC480     - 06/06/92

UMC480     - 11/11/92

UMC480B    - 11/11/92

UMC491C    - 04/04/93

UMC491f    - 04/04/93

UMC498     - 07/25/94

Ver1.00    - 06/06/92

Ver1.01    - 06/06/92

VIAEISA    - 06/06/92

VIPER      - 10/10/94

10/01/1995