asdfg  FirmUp™ Applet

Firmware Update Procedure, Tips and Tricks for Microcom FlashROM Modems

Revised 08/02/96

Technical Support		(617) 551-1313 Monday-Friday 8:00am - 7:00pm EST

Microcom Inc.

500 River Ridge Drive

Norwood MA USA

FirmUp now features enhanced Windows95(tm) compatibility for both  PCMCIA and Parallel Modems.

FirmUp will automatically switch to  serial mode until the firmware upload process is complete.

No additional configuration is necessary for notebook computers under Windows95(tm).

The following Microcom Modems are Flash ROM Upgradeable:

DeskPorte Fast Plus      	TravelPorte Fast

DeskPorte Fast          	TravelCard 28.8 P

Deskporte Fast EP          	TravelCard Fast

DeskPorte Fast 28.8 P	HDFast

NOTE:

External Parallel Modems must be upgraded by Serial Connection.  Firmware can NOT be upgraded through the Parallel port.

Firmware files for all Microcom FlashROM Upgradeble Modems are available at the following locations:

WWW:	http://www.microcom.com

FTP:  	ftp.microcom.com

Modems: /pub/modems/firmware/domestic/   [USA]

/pub/modems/firmware/internat/   	[International]

HDMS:   /pub/hdms

BBS:   	(617) 551-4750

Domestic Modems:     	Conference (2)

International Modems:	Conference (13)

HDMS:                	Conference (3)

Basic Installation Procedure for the Microcom FirmUp(tm) Applet

1. Obtain the current firmware file for your product.

2. Create a directory for the FirmUp program files on your hard drive.

Example: MD c:\firmup)  Copy all of the included files to this directory.

- Install Procedure:

For Windows 3.xx:

Create a program item by selecting File/New

Description: Firmup

Command line: C:\Firmup\Firmup.exe

Working Directory: C:\Firmup

Click on Change Icon.

Double click the icon

Click OK

For Windows 95:

Open Explorer and change to the c:\firmup directory.  Create a shortcut for firmup.exe and drag to the  location of your choice.

4. Update Procedure

Execute FirmUp by double clicking on the icon.  Select the correct  COM port and modem type.  Click on the [INFO] button. If you receive a "Modem not Found" error message, recheck your COM port

selection.

Now select the correct radio button for your Microcom modem product.

Click on the [Update] Button.  A file selection dialog box will appear.

Select the correct path to the firmware file.  Example: c:\firmup\M0853318.DPF

Now click on the [OK] button and the update process will begin.

What to do if the Firmware Update Fails:

Windows 3.11 Flow Control Mismatch:

Microcom Modems are configured to use hardware (RTS/CTS) flow control by default.  Windows 3.1x default settings are XON/XOFF flow control.  You can optimize your port settings by going into  Main -> Control Panel -> Ports -> COMn .  Change the Flow Control setting from XON/XOFF to "Hardware" on the drop down selection box.

Use a Slower Port Speed:

Windows 3.11:

Select a Port Speed of 9600 bps or lower in  Main -> Control Panel -> Ports -> COMn

Windows 95:

From the START Menu, select Settings ->  Control Panel -> Modems Icon -> Modem  Properties Tabbed Page ->  Advanced Settings. Change Serial Port Speed to 9600 bps

Regarding 16550 UART Serial Support:

The TravelCard Fast and TravelCard 28.8 P have integral 16550 support.

The firmware update procedure for all serial-connected Microcom modems is greatly facilitated if the serial port is supported by a 16550 or equivalent buffered UART chip.

The actual main code file for Microcom FlashROM upgradable  products is an ASCII Hex Dump file.  Each line of the firmware code is protected by a checksum.  In the event that a bit is dropped, the checksum will fail.  The 16 byte buffers in a 16550 or equivalent UART chip will greatly reduce the  possibility of a checksum failure due to dropped bits.

Shut Down all other Processes while Updating your Firmware:

FirmUp will have a greater possibility of success when it does not  have to compete with other applications and processes for interrupt service.  We recommend that nothing else should be running for the duration of the update process.

Reset Factory Defaults for Deskporte Fast - All Models:

Experience has shown that the update process may be eased by  resuming a factory default configuration for the DeskPorte Fast,  DeskPorte Fast+ and the corresponding rack mount models. This can be quickly accomplished by switch setting.  Open SW2-4 and turn  the modem off and on.  Leave the switch open.  Proceed with the  update procedure.  Restore the switch to the closed position and  reconfigure the modem.

How to Flash Update Modem Firmware with Terminal Emulation Software:

The following terminal emulation packages have been tested and found  to be capable of supporting this procedure:

CrossTalk for Windows and DOS        	TE/2 for OS/2

Procomm Plus for Windows and DOS    	HyperAccess for OS/2

MicroPhone Pro for Windows           	ZOC for OS/2

QuickLink II for Windows and DOS     	LiveWire for OS/2

Carbon Copy 6.1 terminal mode

Qmodem Pro for Windows and DOS

Telix for Windows and DOS

{COMMO} for DOS

Term90 - Symantec Norton Commander 4.0

HyperTerminal - Windows 95

Basic Steps for Terminal Emulation Software:

Set Serial Port Speed in Software to 9600 bps

Set Flow Control in Software to Hardware Flow Control (RTS/CTS)

If your Software supports only Software Flow Control send AT\Q1 command to modem.

(XON/XOFF Flow Control)

Set ASCII Transfer Options in Software:

Character Pacing  = 		0

Delay between Linefeeds =	0

Send AT^H <enter> to modem to begin Update Process

Immediately begin an ASCII Upload of proper main code firmware file to your Microcom modem.

There will be no screen output during the actual update process.

A successful upload will echo the message "MODEM FIRMWARE UPDATE COMPLETE" to  your terminal screen.

If you receive any error messages, refer to the "What to do if the Firmware Update Fails" section above.

The upload process should take three to seven minutes to complete.

Send AT%V1 <enter> to check current firmware revision level

-end-