Last modified: July 11, 1996

Prestige 2864I PPP/MP Interoperatibility Test Matrix

This test matrix was generated with the results from the MP Interoperability tests that were conducted during the last week of May.  The list consists of all the products that we were compatible with along with some of the problems that we encountered.

| Company | Product | Comments |
| --- | --- | --- |
| 3Com | Impact |  |
| 3Com | Access Server | **CHAP problem |
| ADC Kentrox | Pacesetter |  |
| SAT | Sagem ISDN PPP |  |
| Ascend | MAX 400 |  |
| Gandalf | Xpresstack BRI |  |
| Gandalf | 5242 | *problem with no IP address provided in the IPCP |
| U.S. Robotics | NetServer - Total Control |  |
| U.S. Robotics | Courier I-Modem |  |
| U.S. Robotics | Allegra BRI for NetWare |  |
| U.S. Robotics | Sportster PCTA |  |
| Motorola | BitSurfer Pro |  |
| Diamond | Netcommander |  |
| Microsoft | ISDN Accelerator 1.1 |  |
| Bay Networks | UK |  |
| Develcon | Orbitor 3000 |  |
| Cisco | Pro 753 |  |
| Cisco | 2503 | Cisco had a problem in answering the 2nd call. Their MP representative was not present for further investigation. |
| NetManage | Chameleon |  |
| Frigate | NetBlazer | This company represents Telebit |
| Novell | MPR | **CHAP problem |
| Eicon | INT |  |
| Shiva | Integrated 200 - Shiva UK |  |
| Shiva | Shiva PPP |  |
| Shiva | Lan Rover |  |
| Shiva | LTD |  |
| Flowpoint | Flowpoint 100 |  |
| Sun | SunISDN |  |
| DEC | RouteAbout Access | **CHAP problem |
| SBE | Netxpand SOHO |  |
| FTP | Onnet32 |  |
| Cascade | Hyperpath | *problem with no IP address provided in the IPCP |
| ACC |  | *problem with no IP address provided in the IPCP |

*These devices did not provide an IP address during the IPCP negotiation.  In this situation, we originally dropped the call for security enforcement.  We changed the code to interoperate with these vendors.  Now, we will not drop the call, and we will use the IP address configured in menu 11 to add a route to that remote node.

**The CHAP packet size was too large.