-I-

MARNAV

About Marnav-In the early 1980's, when LORAN A was taken out

of service, many mariners needed a way to  convert  LORAN  A

data  to  the new LORAN C delays.  To meet this need, MARNAV

was written by C & E Associates  to run on the TRS 80 series

of computers.  In addition, early LORAN C receivers did  not

present   data   in  a  Latitude/Longitude  format.   Marine

navigation from one  point  to  another was rather difficult

since, unlike geographic coordinates, LORAN coordinates  are

hyperbolic  position lines.  Most LORAN C users wanted their

data in  geographic  format  to  simplify  their navigation.

This was especially true when the LORAN system  was  out  of

service for one reason or another.

After  several  years,  LORAN  became  available  for use in

aircraft.  At that  time,  only  the  users manual furnished

LORAN data for airports and not for all  of  them.   It  was

easy  to get latitude/longitude data for other airfields and

navigation  waypoints  so  Marnav  was  being  used  in  the

aviation community to make such data available.

In the early nineties,  the  Global Positioning System (GPS)

started to replace  LORAN  C.  Equipment  became  much  more

reasonable in price and more and more mariners and fishermen

were  installing  the new, more accurate, equipment.  We now

had a new problem; Most  LORAN  C users had large amounts of

data in LORAN C format and now needed to  change  that  data

back to GPS format.

In  the meanwhile, Marnav had been upgraded several times to

run on IBM compatible PC's.  Even though it is a goal of any

programmer to make  his  programs  easy  to  use, the entire

field of navigation, LORAN, GPS, etc.  is  technically  very

complicated.  and  writing  user  friendly  software becomes

doubly difficult.  In  Version  4.5,  we  have tried to make

program operation more  understandable.   Most  screens  now

include help blocks on the screen and in some cases the user

is  guided through especially difficult material.  The menus

have  been  changed  to  pull-down  menus  and  many  of the

operations are called directly from the main menu.

There is no difference in the shareware and  the  registered

version  of  Marnav.   The shareware version is shipped with

the West Coast LORAN C chain so the user may try all program

features to make  sure  the  program  does what needs doing.

The data for other chains may be entered from data available

in any LORAN instruction book, or is available from the U.S.

Coast, Department of Transportation at a nominal cost.   The

necessary  routines  are  provided with the program to enter

this data or,  to  add  new  chains  which may be announced.

When you register Marnav, in addition to a  printed  manual,

you  will  also  receive the data files for all of the LORAN

chains in  the  world  so  you  do  not  need  to enter them

yourself since this can be an error prone effort.

The following constants were used for all algorithms in  the

Marnav program.  The signal propagation velocity used is the

propagation  velocity  in free space, 299,792,458 meters per

second;  Sea  water  conductivity  is  5.0  mhos  per meter;

Angular  conversion  is  0.01745329251993889   radians   per

degree.  All calcu lations and data are in double precision.

Calculations necessarily assume an all water path.

In  addition  to  the  LORAN  C  to  GPS and vice versa data

conversion, the  program  provides  the  ability to navigate

from one position to another where either or both  positions

may  be  specified  in  either  LORAN  C or GPS coordinates.

Marnav also calculates relationships between points in terms

of Speed, Time and  Distance  in  either Nautical or Statute

miles and knots or miles per hour.  If the Hard Copy feature

is turned ON, a  hard  copy  record  of  all  operations  is

printed  as  you  proceed.   The  LORAN  data  bases  may be

created, displayed, edited and  printed  as required.  It is

possible to compensate for propagation  anomalies  providing

position calculations accurate to several yards!

Program  Files-There  are  currently thirteen LORAN-C chains

world  wide.   The  necessary  data  for  each  chain  is in

contained in files named for the Group  Repetition  Interval

(GRI).   All  thirteen  of  these files are shipped with the

registered version of  Marnav.   In  addition  to these data

files, one file is maintained to indicate the active  chain.

(The  chain  you are currently using).  When you first bring

up Marnav, it will read this  active chain file and then the

data for the indicated chain.   It  is  necessary  that  the

program  and  all  of  the  data  files  be kept in the same

directory since the  program  looks  for  the  data files in

whatever directory you were  in  when  you  brought  up  the

program.  The active chain is indicated on the Menu screen.

-II-

Marnav Menu

Menu  selection  is relatively standard for pull down menus.

Select your choice  of  columns  using Left/Right arrows and

the menu items by highlighting that item using  Up  or  Down

Arrow  keys.   You  may also select the item by entering the

letter  designation  for  that   item.   Each  of  the  menu

selections will  be  discussed  in  the  remainder  of  this

documentation.

-III-

LORAN-C  to  GPS

The  term  GPS  may  be unfamiliar to some LORAN users.  The

United  States  government  has  established  a  network  of

navigational  satellites  which  provide  extremely accurate

geographic positioning.  The accuracy of the basic system is

so reliable that it is feared that a potential  enemy  could

use  the  data  to  position intercontinental missiles.  For

this reason, there is  an  error  transmitted which causes a

positional error of several hundred feet.  A number of error

correction stations provide  short  range  error  correction

information.   These  stations  may be shut down in times of

emergency.  At any  rate,  this  system  has been called the

Global Positioning System; Hence the abbreviation GPS  which

will be used throughout this documentation.

The  first  item  in  the  column labeled Data Conversion is

LORAN-C to GPS.  The  title  is  self explanatory.  When you

choose this item, the data entry screen  will  be  displayed

with  the  cursor  at  the  entry  point for the first LORAN

delay.  Enter two delays  for  the location in question.  If

they   are   valid,   the   program   will   calculate   the

Latitude/Longitude coordinates for that location.   It  will

then  calculate  the  delays  for  any  other delays in that

chain.  Some chains have  only  two secondaries, others have

three or four.  If you are an  experienced  LORAN  user  you

know  that  in  some  parts  of the service area for a given

chain the  signal  strength  and/or  the  orientation of the

delay lines on the chart are such that  certain  delays  are

somewhat  more  useful  than  others.  It is often useful to

have the delays for each secondary in a chain as a matter of

record for each point.

The Latitude/Longitude is expressed in three formats:

Degrees______________________30.2700

Degrees, Minutes_____________30 16.200

Degrees, Minutes, Seconds____30 16 12.0

The first of these is

generally used on large scale charts and maps, the second is

generally used on small  craft  charts  and the last on very

local geodetic  survey  maps.   The  conversion  is  a  real

nuisance  when  you  are  trying  to locate a spot on map or

chart.  This is also a very useful feature of the program if

you need to convert data from one format to another.

If the data you have  entered  is  an improper value for the

normal service area of the chain you will get a message:

No Useful Solution Found-Check Data

More often than not, there has been a mistake  in  the  data

entry.   If calculated points are a long way from the normal

service area for the chain,  you could not receive the loran

signals anyway.  In addition,  if  the  entered  delays  are

unreasonably far from the approximate latitude and longitude

shown on the screen you may also get the same message.

Calculations  begin  at  the latitude and longitude shown as

approximate coordinates.  If the  location  of interest is a

long way from them the program solution may be slow.  Marnav

works much faster if the approximate coordinates are  within

2  degrees of the actual latitude and longitude.  This is on

the order of one hundred  or  more  miles  so it is rarely a

problem.  These values may be edited in the  data  base  and

also  are  automatically updated for subsequent calculations

in the same area.

The propagation velocity constants  used by the program have

been determined for free space over water.  Minor  anomalies

occur  when  there is considerable land area between you and

the delay transmitters.  The result is that the LORAN delays

for a given point  today  will  be extremely close next time

you use them to define the same point.  Put another way, the

repeat ability of  LORAN  delays  is  extremely  high!   The

absolute     relationship     between    calculated    Lati-

tude/Longitude and LORAN  delays  however  may  have a small

error of several hundred feet  in  some  cases.   In  normal

navigation,  this  would not be a problem but there are many

applications  where  more  accurate  agreement  is required.

Marnav allows the use of Correction Factors to  offset  such

errors.  This will be discussed later.

-IV-

GPS  to LORAN-C

This  operation is opposite to that just discussed.  In this

case, the latitude and longitude of the point of interest is

entered and all of the  secondary  delays for that chain are

calculated.  Data may be entered in any of the three formats

discussed above.  Enter the data in any format and  it  will

be  converted to the other two as well.  For example, if you

enter 30/16.2 it will be  converted to the three forms shown

above.  The delimiter  between  degrees  and  minutes  shown

could  have  been  a  space,  a comma or, as shown a forward

slash.

You should be aware that  just because Marnav indicates that

a delay should have a certain value, is  no  assurance  that

the signal strength of that delay will be great enough to be

useful.   If the LORAN is able to lock on the secondary, you

will be able to use it,  otherwise  you will have to use one

of the others.  In general, the part of the service area you

are in will generally determine the secondaries  you  should

use.  There are also times when the equipment for one of the

secondaries  has  been shut down for service.  Those are the

times when having  all  of  the  secondaries  listed is most

useful.

Anyone using LORAN should be aware that for any  two  delays

there    are    geographically   two   intersections.    The

mathematical reason for this  is  that these delay lines are

hyperbolas and as such will intersect twice.  This may  seem

to  be a drawback to the use of LORAN, but in general, these

intersections are widely  separated  and  if the approximate

latitudes and longitudes shown on the screen  are  within  a

few  hundred  miles, the LORAN and Marnav will solve for the

right one.  At any  rate,  the  coordinates of the wrong one

would be so far away that  one  could  not  readily  mistake

them.

When  latitude  is  used  in  geographic calculations, North

latitude  is  normally  taken  as  a  positive  value.   For

Longitude,  West  latitude  is  normally  a  negative value.

Since Marnav is usually used in the same earth  quadrant  it

lets  you choose which quadrant of latitude and of longitude

is to be considered  positive.   If  you are using Marnav in

the continental United States it is easier to  define  North

Latitude  and  West Longitude as positive quantities instead

of remembering  to  put  the  minus  sign  in  front  of the

longitude entries.  The quadrants you choose as positive are

shown on the screen and may be changed by editing  the  data

base

-V-

Bearing & Distance

This menu choice invokes the bearing and distance navigation

routines.   The  coordinates  for  either or both of the two

points  may  be  entered   in  either  LORAN  or  geographic

coordinates.   Note  that  LORAN  coordinates  must  all  be

expressed for the active  chain  and  both  points  must  be

defined  in  the  same chain.  If both points are defined by

GPS  coordinates,  the  chain   does   not  enter  into  the

calculation.  It follows then that  coordinates  defined  by

LORAN delays must be in the active chain but the other point

may  be  defined  geographically (GPS) and may be outside of

the service area for that chain!

Initially the cursor will  be  positioned  for a LORAN delay

input; Simply press <Enter> and the cursor will move to  the

geographic  input  position.  The second position is entered

in the same  way  and  the  results  of  the calculation are

displayed near the  bottom  of  the  screen  as  rhumb  line

heading  and  distance  to  and from the two points entered.

All the same comments  concerning  data  entry apply as they

are discussed in earlier sections of this documentation.

-VI-

Speed/Time/Distance

This menu  choice  provides  the  relationship  between  the

distance from one point to another, the speed of the vehicle

and  the time of travel.  Data entrance is extremely simple,

if you enter any  two  of  these quantities the program will

calculate the value for the third.

If the quantities entered are speed and distance,  the  time

will  be  calculated  and  the  time is presented in decimal

hours as well as hours and decimal minutes.  If the distance

is entered in Nautical Miles,  the speed will be calculated,

or, must be entered in Knots.  Conversely, if the  speed  is

entered  in Statute miles then the speed will be calculated,

or, must be entered  as  Miles  per  Hour.  Obviously, it is

also true that if the speed is entered in  Knots,  then  the

distance  is  calculated,  or,  must  be entered in Nautical

Miles and vice versa.

If time is one  of  the  quantities  being entered, the same

formats as discussed previously for degrees and minutes  may

be  used.   For  example, 3.75 hours and 3/45 ( 3 hours & 45

Minutes ) will produce the same result.

-VII-

Display Data

This item, and the next  four  deal with the data base which

defines the characteristics  of  the  LORAN  chain  in  use.

Selecting  Display  Data will present the data for the chain

in use.  In addition to  displaying  the data on the screen,

you may change the active chain and also make a hard copy of

the data for this chain.

The data displayed consists of an accurate position in terms

of Latitude and Longitude of the Master for  this  chain  as

well as each of the Secondaries.  As mentioned earlier, some

chains  have only two secondaries while others have three or

four.  Each secondary transmits  its  signal after a precise

display after the master and this delay is shown along  with

the positional data for the secondary.

Correction  factors  to compensate for propagation anomalies

are also shown.  These are discussed in detail in a separate

section.  In addition,  the  GRI  number  is shown.  This is

more than an arbitrary number since  it  defines  the  Group

Repetition  Interval  which  defines  the  signals  for this

chain.   Obviously  these  positions   and  delays  must  be

extremely accurate or the system is useless!

Also shown are the Quadrants in which Latitude and Longitude

are arbitrarily defined as positive values.  The approximate

latitude and longitude used by the program as initial points

for calculation are also shown.

-VIII-

Edit Data Base

Any of the items discussed above may be  edited  using  this

menu   selection.    It  is  strongly  suggested  that  only

approximate latitude, longitude,  the positive quadrants and

correction factors be edited without extremely  good  reason

and then, only after backups of the data are saved.

The Edit sequence is started by selecting EDIT DATA from the

on  screen menu bar.  You may step through the data field by

using <Enter>, <up-arrow> or <down-arrow>.  Data corrections

are made in the usual way.  When you have finished with your

edits, press <End> to return to the on screen menu.  Changes

made when editing this data  base  are recorded to disk only

when you select WRITE FILE and press <Enter>.

Without leaving the Edit function, you may change the Active

GRI or make a hard copy of the data on the screen.

-IX-

Initial  GRI

The initial GRI is that chain which is active when you start

the program.  Most users do not normally change  the  active

GRI  very often but, at times, changing the location of your

activities makes  it  necessary  to  do  so.   Simply select

Initial GRI from the menu.  This will change  not  only  the

GRI which is active when you start the program but will also

change  the GRI which you are currently using.  As mentioned

previously, it is also possible  to  make this change in the

Display and Edit modes.

-X-

Correction Factors

LORAN  Calculations  and  their  accuracy  depend   on   the

propagation  velocity  of radio waves.  Unfortu- nately, the

propagation velocity is not exactly  the same over all types

of terrain so the delay time to a given point  may  be  very

slightly  different than it is to another point which is the

same distance from the station.  While these differences are

extremely  small,  the  difference  can  lead  to  errors of

several hundred yards when the path from the station to your

position crosses large land masses.

If your concerns are simply with getting to the same spot as

you were at some previous time, then  this  error  will  not

concern  you  since  the repeatability of the LORAN data for

that spot is extremely  good.   The  problems occur when you

have the GPS position for a  given  spot  and  you  want  to

convert to LORAN coordinates, or, in the opposite case, when

you want to convert your LORAN data to GPS coordinates.

In  most  cases, boaters and fisherman operate in relatively

small areas often as small  as  ten  miles  or so on a side.

Unless there are large geographic differences near the area,

the propagation errors will probably  be  fairly  consistent

throughout the area.  In that case, you may add a correction

factor  to  your  data  base which will compensate for these

propagation anomalies.

In Marnav 4.5 the correction factor routine has been written

to evaluate and  enter  this  data  quickly  and easily.  In

order to  provide  the  necessary  data  to  evaluate  these

errors,  you  will need accurate data for a point in or near

the  area  of  interest.   Accurate  geo-  graphic  data  is

available from a  number  of  sources.   Newer marine charts

have accurate positions for buoys and markers  or,  you  can

get accurate geographic data from a GPS receiver which has a

correction    receiver   to   compensate   for   the   error

intentionally put into the GPS  data as transmitted from the

satellites.  You may also get accurate  data  from  Geodetic

survey maps in which positions along the shore are described

using survey data.

The data must be for a place where you can locate your LORAN

to  get  LORAN  delays for that spot.  You should try to get

data for all of the secondaries in the chain.  In many spots

this will not be possible because some secondaries will have

very weak signals but try to get as many as you can.

To enter  correction  factors  into  your  data base, select

Correct.  Factors from the  menu  and  follow  the  Onscreen

instructions.   The factors will be automatically calculated

and entered into  the  data  base.   From  then on, they are

automatically included in the calculations and,  unless  you

move  to an area far from where they were determined, you no

longer need concern yourself about them.

-XI-

Adding A Loran Chain

It is unlikely that any new LORAN chains will be constructed

since the LORAN system will  gradually be phased out as more

and more users get and use GPS equipment but you may wish to

add data for chains not in your files.  The U.S. Coast Guard

makes available the exact geographic location of the  Master

and  each  of  the  secondaries  as well as the transmission

delays for each of  the  secondaries.  Enter these data very

carefully  using  as  many  decimal  places  as   are   made

available.   When  all the data has been entered, select the

Write File sub-menu option  on  the  data  screen and a file

called GRIxxxx.LRN will be created where the  xxxx  are  the

numbers which define the Group repetition rate which will be

defined along with the other Coast Guard data.

-XII-

Hard Copy

The  Hard  Copy choice on the menu will toggle the Hard Copy

switch  Off  and   On.   When   On,   each   calculation  is

automatically printed  on  your  system  printer  for  later

reference.   This feature is especially valuable when cruise

planning or calculating your navigation data before starting

your journey.  The GRI  data  bases  may also be printed but

that printing is done when you select the PRINT DATA  choice

from the on-screen menu during the data operations.

Shareware

C & E Associates has chosen  to use the Shareware concept to

distribute this program.  This is good for  us  in  that  it

contains  many  of  the costs associated with a retail sales

effort.  It is good for you  since  it allows you to make an

intelligent buying decision by trying the  program  to  make

sure  it does what you want before you spend your money.  It

does however bring with it  a  legal and moral commitment to

buy (Register) the program after you have tried it and found

it to be to your liking.  The authors of  Shareware  receive

NO  money  from the distribution of their efforts until you,

the user, register it.  If  you believe the effort that goes

into writing a program such as this deserves the thirty-five

dollar registration fee then please register by sending your

check along with  the  form  included  at  the  end  of  the

documentation.

No program features have been "held back" until you register

the  program.  When you register, you will receive a copy of

the latest version of  the  program  as well as discounts on

any future program upgrades.  In addition, you will be  kept

informed  about  several  other  programs  in development on

related subjects.   Program support will be provided to both

non-registered and  registered  users  using Compuserve Mail

addressed to Edward J. Panner, (72717,1612) or  by  U.S.Mail

addressed to Edward J. Panner at the registration address.

This  program  is produced by a member of the Association of

Shareware professionals (ASP).  ASP  wants to make sure that

the shareware principal works for you.  If you are unable to

resolve a shareware related problem with  a  ASP  member  by

contacting  the  member  directly,  ASP may be able to help.

The ASP Ombudsman can help  you resolve a dispute or problem

with an ASP member, but does not provide  technical  support

for members' products.  Please write to the ASP Ombudsman at

545  Grover  Road,  Muskegon,  MI 49442 or send a Compuserve

message via Compuserve Mail to ASP Ombudsman 70007,3536.

Additional information about the legalities of Shareware may

be found in the appendix to this documentation.