RELEASE NOTES

for

Version 2.09

of the

Configurable Initialization Program

and its companion

Interactive Configuration File Editor.

In the notes below where Q-PC is used it refers to either a Q-PC, Q-MCA or a

Q2000 board.

QINIT

NAME

QINIT - configurable Q-PC board initializer.

SYNOPSIS

QINIT [-a] [-b] [-f font_xy] [-h] [-i screen_initial_xy] [-o po_ctrl]

[-p] [-q qpc_no] [-s] [-v] [config file]

DESCRIPTION

All the above parameters to QINIT are optional.

QINIT's primary and default operation is to initialize the first Q-PC

board in your computer with the set of parameters contained in the

configuration file, and inform the Q-PC hardware characteristics

driver of the board's hardware characteristics.

If the Q-PC hardware characteristics driver is not resident when a Q-PC

board number greater than one is specified or when a Q-PC hardware

characteristic differing from the following defaults:-

Q-PC board base address (0300H)

Q-PC board interrupt request number (IRQ) (10)

Q-PC board DMA channel (7)

Q-PC board memory size X (2048)

Q-PC board memory size Y (1024)

Q-PC board memory fold (1024)

Q-PC board planes (0x0f)

is specified, the board initialization will be aborted.

The initialization sequence is :-

1. Reset the QPDM.

2. Initialize the QPDM monitor parameters.

3. Set up the Q-PC hardware characteristics driver.

4. Set the default QPDM drawing modes.

5. Clear all of the video memory.

6. Initialize the palette.

7. Load a 9 by 14, a 11 by 20 & a 12 by 24 font into the

off screen memory.

8. Display the Datapath company logo and a

`initialization completed statement'.

If a file extension is not specified for the `config file' QCF is

assumed. If your `config file' has not got a file extension you must

enter a '.' after the file name. The file extension BAK is not allowed.

If `config file' is omitted, the default file QPC.QCF is used. You may

copy the configuration file for your `favourite' monitor to this file

to save typing. When no disk drive or path has been specified as part

of the `config file' specification, the following actions are taken:-

1) the configuration file is looked for in the current

directory.

2) if the file wasn't found then

a) if the environment variable QCFDIR is set, the path found

there is used to specify the directory where the configuration

file should be found.

b) otherwise the directories specified in the PATH environment

variable are searched for the configuration file.

The other options allow you to do the following :-

-a allows the Q-PC hardware characteristics driver to be

`avoided' for the storage of memory information ie if this

option is used the Q-PC hardware characteristics driver need

not be present in memory even if the board being initialized

has a memory configuration to the default.

THIS OPTION IS INTENDED FOR DEBUGGING PURPOSES ONLY AND SHOULD

NOT GENERALLY BE USED.

-b turns on backwards compatibility with versions of

QINIT/QCONFIG prior to 2.00.  It may be required if

drivers/utilities issued prior to 1st June 1990, by Datapath,

or non-Datapath drivers are used.  See `technical information'

later on if you want to see what it affects.  It is ignored in

the Q2000 versions of QINIT/QCONFIG.

-f font_xy, where font_xy is either:-

1) a QPDM absolute X, Y address pair (entered as 1280,0 or

"1280 0" for example), specifies the top left hand corner of

the area where the fonts should be stored.  They are stored,

by default, just off the screen at the top right for memory

configurations except F, G & H where they are stored just off

the screen at the bottom left.

2) the letter 'r', specifies that the top left hand corner

of the area where the fonts should be stored, should be just

off the screen at the top right.

3) the letter 'b', specifies that the top left hand corner

of the area where the fonts should be stored, should be just

off the screen at the bottom left.

4) the letter 'x', specifies that the fonts should be stored,

at the top right as far over in x as possible.

5) the letter 'y', specifies that the fonts should be stored,

at the bottom left as far down in y as possible.

6) the letter 'v', specifies that the fonts should be stored,

in the correct place for the Vutrax driver.

-h performs a hard reset of the Q-PC board. A hard reset is

items 1 and 2 of the initialization sequence described above.

Note that -s has precedence over -h therefore if they are both

specified only the -s will be executed.

-i screen_initial_xy, where screen_initial_xy is a QPDM

absolute X, Y address pair (entered as 1280,0 or "1280 0" for

example), specifies the top left hand corner of the screen.

The default coordinates for the top left hand corner of the

screen when this option isn't specified, are 0,0.  This option

is provided so that off screen memory may be examined on a UNIX

system.  It is designed to be used with the -h option so that

the memory isn't altered.

-o po_ctrl, where po_ctrl means palette overlay control and can

be `on', `off' or a number, controls the overlaying of the

bottom half of the video memory or which planes of the video

memory are to be displayed by the palette.  Note that this

option is only effective if the palette is initialized.

-p allows you to specify that only the palette should be

initialized. Hence the board parameters, monitor parameters

and initialization completed statement need not be present in

the `config file'. Note that if -p is specified with -s, the

palette will be initialized with QINIT's default palette data,

not the palette data in a 'config file'.

-q qpc_no, where qpc_no is a base 10 number, is used to specify

which Q-PC board in the computer that the parameters in the

`config file' refer to, and hence for which board the hardware

characteristics from the configuration file should be stored in

the Q-PC hardware characteristics driver.

-s performs a soft reset of the Q-PC board. A soft reset is

items 4, 5 and 7 of the initialization sequence described above.

Note that if -s is specified a `config file' is never read, even

if one is given.

-v causes the version number message to be issued. The board

is not initialized if this option is given.

The -f & -o options should only really be used when instructed to do so

in the installation instructions of a Q-PC driver.

The -h, -s & -p options do not inform the Q-PC hardware characteristics

driver of the boards hardware characteristics but rely on the

characteristics previously set into the driver.

QINIT returns the following exit codes:-

0 - File parsed and Q-PC graphics board initialized O.K.

1 - Q-PC graphics board initialization completed incorrectly.

2 - A invalid file specification was given on the shell command

line.

3 - The file specified could not be found.

4 - There were invalid options on the shell command line.

5 - There was an erroneous parameter in one of the obligatory

sections found in the file.

6 - The Q-PC hardware characteristics driver was not resident

when a Q-PC board number greater than one was specified or

when a Q-PC hardware characteristic differing from the

following defaults:-

Q-PC board base address (0300H)

Q-PC board interrupt request number (IRQ) (10)

Q-PC board DMA channel (7)

Q-PC board memory size X (2048)

Q-PC board memory size Y (1024)

Q-PC board memory fold (1024)

7 - The Q-PC board number specified was reported as invalid by

the Q-PC hardware characteristics driver.

8 - The Q-PC board was found to be incorrectly initialized when

trying to perform a soft reset operation or palette

initialization.

9 - The version message option was given therefore QINIT gave

the requested version message then exited.

EXAMPLES

QINIT cotron64

-- initializes according to parameters in COTRON64.QCF

QINIT

-- initializes according to parameters in QPC.QCF

THE CONFIGURATION FILES

The configuration files end in .QCF (for Q-PC ConFiguration). You can edit

these files with the Interactive Configuration File Editor or with a text

editor IF YOU KNOW WHAT YOU'RE DOING, using the text editor being the harder

of the two.

Note: The monitor vertical parameters in the configuration file relate to the

field and so when using interlacing they are half the values for the frame.

Configuration files, for the supported Q-PC board and monitor combinations, are

built using the utility QCFBUILD.

QCONFIG

NAME

QCONFIG - the interactive configuration file editor program.

SYNOPSIS

QCONFIG [-a] [-b] [-q qpc_no] [config file]

DESCRIPTION

All the above parameters to QCONFIG are optional and have the following

functions :-

-a allows the Q-PC hardware characteristics driver to be

`avoided' for the storage of memory information ie if this

option is used the Q-PC hardware characteristics driver need

not be present in memory even if the board being initialized

has a memory configuration to the default.

THIS OPTION IS INTENDED FOR DEBUGGING PURPOSES ONLY AND SHOULD

NOT GENERALLY BE USED.

-b turns on backwards compatibility with versions of

QINIT/QCONFIG prior to 2.00.  It may be required if

drivers/utilities issued prior to 1st June 1990, by Datapath,

or non-Datapath drivers are used.  See `technical information'

later on if you want to see what it affects.  It is ignored in

the Q2000 versions of QINIT/QCONFIG.

-q qpc_no, where qpc_no is a base 10 number, is used to specify

which Q-PC board, in the computer, the parameters in the `config

file' refer to and hence for which board, the hardware

characteristics from the configuration file should be stored in

the Q-PC hardware characteristics driver.

`config file' is a configuration file specification and is used

to specify the configuration file you initially wish to edit

with QCONFIG. If a file extension is not specified for the

`config file' QCF is assumed. If your `config file' has not got

a file extension you must enter a '.' after the file name. The

file extension BAK is not allowed.

QCONFIG is entirely menu driven and its operation should be self

explanatory to its intended users - persons familiar with the

operation of monitors and the Q-PC boards.

Note: The Initialization Completed Correctly Statement is only saved

in the configuration file if it is altered from the default of

'Initialization Completed', similarly only the palette colours that are

altered from their default values, which can be seen by looking at the

Palette Colours menu when you haven't read in a configuration file, are

saved.

Once you have created a configuration file with QCONFIG you may insert

a comment, about the configuration described by the parameters in the

file, at the start of the file with a text editor.

QCONFIG returns the following exit codes:-

0 - File parsed and Q-PC graphics board initialized O.K.

1 - There were invalid options on the shell command line.

TECHNICAL INFORMATION

The QPDM's SXT register is set to half what it should be to avoid funnies with

large values, unless the -b option is used with non Q2000 versions of

QINIT/QCONFIG.  A 0 in the SYT register means 4096.

The default QPDM drawing modes set are :-

viewport 0,0 = screen bottom left

scale factor = unity ie no scaling

activity bits = 0x0f

listen bits = 0x0f

clipping, picking and Logical PEL are disabled

For old style Q-PC configuration files with TYPE parameters:-

The palette overlay is turned on for board types 1 & 2 when the on

screen Y resolution isn't greater than the memory fold.

For Q-PC configuration files with PLANES parameters:-

The top 4 bits of the plane mask are used to turn on the palette overlay

planes when the on screen Y resolution isn't greater than the memory

fold.

The default memory configuration is that of a 1M QPC2

ie a short 1M Q-PC/Datapath 1024 board which is as follows:-

2048, 1024,		/* memory size */

512,				/* memory fold */

0,					/* board type */

0,					/* board flags */

0x0f,				/* planes fitted - mainly for monochrome card */

Ian Partridge

V2.09 29 NOV 91

DATAPATH Ltd

Alfreton Road

Derby

DE2 4AD

ENGLAND

Tel. (0332) 294441

Fax. (0332) 290667