DATAPATH Q-PC Hardware Characteristics Driver Controller

SUMMARY

=======

QPCHWCTL is the Datapath Q-PC hardware characteristics driver controller.

Its purpose is to display and change the hardware characteristics stored in

the Q-PC Hardware Characteristics Driver.  Its main use is in trouble shooting.

The Datapath Q-PC Initialization program QINIT, and the Datapath Configuation

File Editor QCONFIG, are used to set the hardware characteristics in normal

use.

USAGE

=====

Displaying information.

qpchwctl

- display the settings for the first qpc

qpchwctl -q <number>

- display the settings for qpc <number>

qpchwctl -a

- display the settings for all the qpcs known to QPCHW

The standard display format is as follows.  It is hopefully self explanatory.

Q-PC	BA	IRQ	DMA	FLAGS	XMEM	YMEM	FOLD	PLANES

1	0x300	10	7	0x1	2048	1024	1024	0x0f

If the qpc number does not have information set, the qpc number is out

of range, or the hardware characteristics driver is not installed, an

appropriate message is printed out.

Changeing information.

qpchwctl -q <number> -b <base address> -i <irq> -d <dma> -x <xsize> -y <ysize>

-f <mem fold> -p <planes>

Set the selected characteristic(s) for qpc <number>.

If -q <number> is ommitted, the first qpc is used.

The given characteristics are merged in with the existing ones.  If no

characteristics are set, then the default values (see qpchw.doc) are set for

characteristics not specified on the command line.

The resulting characteristics are displayed.

SUPPORT

=======

Please report any problems to

Mark Ferguson

DATAPATH Ltd

Alfreton Road

Derby

DE2 4AD

V2.01 24 Oct 1990