Notes on single screen operation and the DATAPATH Single Screen Utilities..

Monitor Configuration:

In single screen operation, a multisync monitor is required.

Configuration information for NEC, Hitachi and Eizo multisync

monitors is provided.  Multisync monitors seem very

particular about the sync timings.

Utilities:

qtype.exe	- display the board type and its current settings

seeqpc.exe	- disable VGA and enable Q-PC, single screen

seevga.exe	- disable Q-PC and enable VGA, single screen

seeboth.exe	- enable Q-PC and VGA, dual screen

qmode.exe	- set the operating mode of the board

These utilities are applicable to any type of Q-PC graphics board.

Some functions are not supported by all boards.  You will be told

if your board does not support a particular function.

All these utilities take the following option:

-q <qpc_number>

This selects the first, second, etc, Q-PC display in your system.  Note

that the hardware characteristics driver must be installed for this

option to work correctly.  It is unlikely that more than four Q-PC

boards will ever be supported.

Apart from QMODE, these utilites have no parameters.

QMODE takes one parameter, an integer representing the new mode.

For a QPC board, the values are

1 == DMA on DREQ,	2 == external enable,

4 == swap memory map,	8 == Q-PC enable.

These values may be added together, to change several bits at once.

For other boards, some bits change meaning.  You will be able

to deduce this from the messages printed out.

The mode is set.  The board powers up in mode 0.

SEEQPC, SEEVGA, SEEBOTH only work if the single screen link

(VGA feed through) is fitted.

SEEBOTH is useful for enabling dual screen operation with out changing

the links on the board.  You have to plug a screen into the VGA video

connector, instead of the Q-PC board.  If you leave the wiring set for

single screen operation, you will get very confused.

Note that the Q-PC board hardware resets to VGA mode.  If you reboot by

ALT-CTRL-DEL you will not hardware reset the board.  If the board was in

Q-PC mode you will not see the boot messages until you run QINIT or

QMODE 0 or SEEVGA.

Mark Ferguson

DATAPATH Ltd

Alfreton Road

Derby

DE2 4AD

ENGLAND

SSU V2.00 11 July 1990