TWIN for Windows 95 1.40

------------------------

Please read the following points before installing the software :-

1: To install the driver use the DISPLAY applet in Control Panel, or right-click

on the desktop and select Properties.  Click on the "Settings" Property Sheet

and then click on "Change Display Type",  click on the "Change" box next

to "Adapter Type" and then click on "Have Disk".  Select the TWIN for

Windows software drive or folder. Windows 95 will now install the driver.

2: The install script modifies the AUTOEXEC.BAT to install the initialisation

program, ORIINIT.EXE or PCIINIT.EXE.  However,  Windows 95 does not

always reboot after installation, YOU MUST ENSURE that the system has

rebooted so that the slave channel is initialised.

3: When you first install the driver you will only be able to select single

screen resolutions. Select any resolution at the desired pixel depth (number

of colours) and restart Windows 95 so that you are using the TWIN for

Windows driver and the slave channel is initialised.

After Windows95 has restarted, you can return to the Display Settings

Property Sheet and set up a twin-screen resolution.

When you select a display resolution, the options displayed are in total pixels e.g.

twin screen 640x480 horizontal arrangement is listed as 1280x480.

As long as you do not change the pixel depth you can change resolution and

number of active screens dynamically, without restarting Windows 95.

4: The Windows 3.11 version of BLACKHOLES does not work correctly with Windows

95. A new version of BLACKHOLES called TWINControl is currently under development.

Known Problems

--------------

Not all of the display driver entry-points are fully implemented in this release.

This means that a few applications may run slowly. e.g. MediaPlayer.

We will implement these interfaces in a future release.

DirectDraw is not supported.

Mouse-Trails are not supported.

DPMS power-saving only works on the master screen, this will be fixed in a

future release.

Bug/Problem reporting

---------------------

Please report problems by Fax (Intl.  +44 (0) 1332 290667) or e-mail to

twin@datapath.co.uk

Datapath Limited

Alfreton Road

Derby

England

DE21 4AD