Diamond Multimedia Systems VIPER SE/PRO Power Management vers. 1.0d

09/26/94

Diamond Display Power Management supports three power management states

defined by VESA  (Video Electronics Standards Association, version 1.0):

STANDBY

SUSPEND

OFF

Power management states in DOS environment can be individually set from 1

to 99 minutes through options of VPROPM (Viper SE/PRO power management TSR

intallation program) as following:

Usage:  VPROPM [option]

Options:

h - Displays this help message

u - Unloads utility from memory

bxx - sets Standby timer in minutes (1 to 99) in DOS environment

sxx - sets Suspend timer in minutes (1 to 99) in DOS environment

oxx - sets Off timer in minutes (1 to 99) in DOS environment

For example:

VPROPM b7 s15 o27 <enter>

Viper SE/PRO will be in STANDBY state after 7 minutes in DOS

environment.  It will be in SUSPEND state 8 minutes after STANDBY

state in DOS environment.  Then in OFF state 12 minutes after SUSPEND

state in DOS environment.

After Viper SE/PRO power management TSR is loaded, you can use VPROPM

to show current power management timer settings in DOS environment.

For example: C:\VPROPM <enter>

Display adapter is set to ON state with input from keyboard or mouse (if mouse

device driver is loaded) in DOS environments.

DISKETTE CONTAINS:

VPROPM.COM     Viper SE/PRO Power Management BIOS TSR 1.0d

INSTALLATION:

1.  Copy VPROPM.COM to root directory C:\.

2.  Modify AUTOEXEC.BAT.  Add C:\VPROPM [option] to set Standby,

Suspend and Off timers in DOS environment.

Viper SE/PRO Power Management BIOS TSR will be loaded with every

system boot, you can change those timer settings later by unloading

it and loading again with new settings.

NOTES:

STANDBY minutes must be <= SUSPEND minutes

SUSPEND minutes must be <= OFF miniutes

LIMITATIONS:

In protected mode application (i.e. AutoCad, Links386 game), DPMS is

not properly supported at this release.

PROGRAMMER'S REFERENCES:

This Viper SE/PRO DMPS BIOS TSR supports following VBE/PM services

(VESA Display BIOS Extensions / Power Management, VBE Sub-function 10h)

as defined in VESA VBE/PM version 1.0 (Feb. 4, 1994):

a. Report VBE/PM Capabilities:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 00h       Report VBE/PM Capabilities

ES:DI          Null pointer 0000:0000h (reserved for future

use)

Output:  AX =           Status (*)

BH =           Power saving state signals supported

1 = supported, 0 = not supported

bit 0         STANDBY

bit 1         SUSPEND

bit 2         OFF

bit 3         REDUCED ON (intended for flat panel)

bits 7-4      reserved for future power control

BL =           VBE/PM Version number (00010000b - 1.00)

bits 3-0      Minor Version number

bits 7-4      Major Version number

ES:DI          Unchanged

b. Set Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 01h       Set Display Power State

BH =           Requested Power State

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

Output:  AX =           Status (*)

BH =           Unchanged

c. Get Display Power State:

Input:   AH = 4fh       VESA Extension

AL = 10h       VBE/PM Services

BL = 02h       Get Display Power State

Output:  AX =           Status (*)

BH =           Power state currently requested by display

00h       ON

01h       STANDBY

02h       SUSPEND

04h       OFF

08h       REDUCED ON (intended for flat panel)

* Status information:

AL == 4Fh:     Function is supported

AL != 4Fh:     Function is not supported

AH == 00h:     Function call successful

AH == 01h:     Function call failed