Orchid Technology                                   VESA BIOS Extenisions v1.21

===============================================================================

The ORCHDVSA.COM is the VESA BIOS Extension Version 1.21 to conform with

VESA standards.  The following standard modes are supported:

Supported VESA Modes

-------------------------------------------------------------

Board             |    100h    101h    102h    6ah     103h    104h    105h

==================|============================================================

Designer VGA	  |		X	X	X

Designer 800      |		X	X	X

ProDesigner       |		X	X	X	X

ProDesigner Plus  |		X	X	X	X

ProDesigner /e    |	X	X	X	X	X	X

ProDesigner II    |	X	X	X	X	X	X	X

------------------|------------------------------------------------------------

Horizontal        |    640     640     800     800     800    1024    1024

Vertical          |    400     480     600     600     600     768     768

Colors            |    256     256      16      16     256      16     256

RAM Required      |    256K    512K    256K    256K    512K    512K   1024K