Date: 10/20/95

Version:1.20 B5

Product: Everest

===========================================================

1. Problem : The panel is hang when VM does the memory 4K page copy.

solved : 1. The bit 6 of SR21 should be set when simulscan is on

program modefied: vddoem.asm, vddint.asm, vddstate.asm,

vddvmmem.asm

2. Enlarge the vertical retrace when VDD finish the pre-

restore the CRTC.

program modefied: vddoem.asm

Date: 10/23/95

Version:1.20 B6

Product: Everest

===========================================================

1. Problem : The TFT panel is hang when simulscan is on.

solved : 1. The bit 6 of SR21 should be disable when single-scan STN

color mode is selected.

program modefied: vddoem.asm, vddint.asm, vddstate.asm,

vddvmmem.asm

2. Problem  :  The setmode 13 will generate screen garbage when the window

dos screen switch to full dos screen.

solved :  1. do 16 times copy 4K memory to physical display buffer

program modefied : vddvmmem.asm

Date: 10/31/95

Version:1.20 B7

Product: Everest

===========================================================

1. Problem : The keyboard will be lock when do ALT+ENTER,ALT_ESC or ALT_TAB

solved : 1. disable the Keyboard before wait vertical retrace.

2. wait one frame of vertical before restore the CRTC.

3. enable the Keyboard after finish the CRTC restore.

program modefied : vddoem.asm , vddstate.asm

Date: 11/07/95

Version:1.20 B8

Product: Everest

===========================================================

1. Problem : Setmode 66 cause screen off in full dos windows

solved : 1. Skip the current VM memeory disable, and set VDD mode flag

to notice mode chang is finish

program modified : vddint.asm vddvmmem.asm

Date: 11/22/95

Version:1.30

Product: Everest

===========================================================

1. Problem : Setmode 66 cause screen off in full dos windows

solved : 1. Skip the current VM memeory disable, and set VDD mode flag

to notice mode chang is finish

program modified : vddint.asm vddvmmem.asm

2. Problem  : Window video picture can't work well when come back from full

DOS screen

solved : 1. save and restore CRTC 3ch

program modified : vddoem.asm vddproc.asm

Date: 12/13/95

Version:1.31

Product: Everest

===========================================================

1. Problem : setmode 64, 65, 66 cause dos window became blank.

solved : bypass the setmode if panel not support.

program modified : vddint.asm

2. Problem : After open several dos window then close all of them, the setmode

13 show error msg 'application violated system integrity ..."

program modified : vddvmmem.asm

3. Problem : Hit CTRL+ALT+DEL on TV mode and you screen out of Sync.

solved : In TV mode, the SR1 clocking mode register has to select 8 dot

clock ( bit 0 = 1 )

program modified : vddsvc.asm

Date: 12/18/95

Version:1.32

Product: Everest

===========================================================

1. Problem : setmode 64, 65 , 66 can't work with new driver B16.

solve : vdd can't program the SR07 and SR2D before

restore screen to plane ( only for 256, 64k color )

program modified : vddvmmem.asm

2. Problem : wall paper happen when press ALT+TAB in full dos screen ( only

happen on 16M color )

solve : use old wait vertical retrace different from 256,64k

color

program modified : vddoem.asm , vddstate.asm

Date: 12/19/95

Version:1.33

Product: Everest

===========================================================

1. Problem : Take out detect setmode 64, 65 , 66, otherwise cause a lot problems

solve : take out module "CheckifModeSupportOrNot"

program modified : vddint.asm