CUPL's Quick Reference Guide

CUPL and CSIM input and output files

���������Ŀ

� Device  �

���������������Ĵ Library ���������������Ŀ

�               �����������               �

�                                         �

���������Ŀ                               ���������Ŀ

�         ������������� .ABS ������������#�         �

.PLD ��#�  CUPL   �������������������������������#�  CSIM   �#�� .SI

�         �����������Ŀ       �      ����Ĵ         �

�����������           �       �      �    �����������

� �   �             �       �      �         �

� �   �             �     .JED     �         �

� �   �             �       �      �         �

.LST #���� �   �             �       �      �         ����# .SO

�   �            .HL      �    .JED

�   �             �       �      �

.DOC #����   �             �       �      �

�� .HEX Ŀ    �       �      �

�    �       �      �

������#����#�������#������#���Ŀ

�      Logic Programmer        �

��������������������������������

.ABS - absolute file needed for simulation

.DOC - documentation file

.HEX - HEX download file

.HL  - HL download file

.JED - JEDEC download file with or without test vectors

.LST - Listing file with errors

.PLD - Logic source file

.SI  - Simulation source file

.SO  - Simulator output with errors

PALASM-TO-CUPL LANGUAGE TRANSLATOR

����������������������������������

The PTOC program converts PALASM source files into CUPL source files.  To

convert one or more PALASM source files into CUPL format, type

PTOC filename1 filename2 ... <RETURN>

For example:

PTOC BUS_CNTL.ASM <RETURN>

produces the CUPL file

BUS_CNTL.PLD

and, if the original PALASM file contained function table information, it

would also produce the file

BUS_CNTL.SI

THE CUPL PREPROCESSOR

���������������������

The preprocessor program operates on the CUPL source file before compiler

operations actually begin.  Preprocessor capabilities include:

String Substitution

$DEFINE argument1 argument2

where argument1 is replaced with argument2 until

$UNDEF argument1

is encountered.

File inclusion

$INCLUDE filename

where filename becomes part of the specification at the compile time.  The

file must be in the current directory.

Conditional Compilation

Portions of the source specification can be compiled or not depending on

whether or not the argument has been defined using the $DEFINE command.

The formats are

$IFDEF argument

... statements ...

$ELSE

... statements ...

$ENDIF

or, if not defined:

$IFNDEF argument

... statements ...

$ELSE

... statements ...

$ENDIF

RUNNING CUPL

������������

In the main menu, select Compile CUPL file.  Select the PLD file that you

want to compile.  A list of CUPL compiler option flags will appear.  Choose

the needed flags for your design.

LOGIC DESCRIPTION FORMATS

�������������������������

A design can be described with a combination of three formats:  state

machine, truth table, or boolean equation.

State Machines

When implementing a design with a state machine, use the following format

Input

���������ķ   Condition   ���������ķ             ���������ķ

����#� State 0 ���������������#� State 1 �������������#� State 2 �

�    ���������Ľ       �       ���������Ľ             ���������Ľ

�         �            #

�����������         output

!Input           variable

Condition

SEQUENCE name_of_sequence {

PRESENT state0

IF input_condition  NEXT state1     OUT output_variable ;

DEFAULT             NEXT state0 ;

PRESENT state1

NEXT state2 ;

.

.

.

}

The simple, flexible syntax allows variations in the IF NEXT OUT or DEFAULT

NEXT OUT formats to properly represent any element of a state machine.

IF          NEXT                    Conditional Next State

IF          NEXT        OUT         Conditional Synchronous Output

NEXT                    Unconditional Next State

NEXT        OUT         Unconditional Synchronous Output

OUT         Unconditional Asynchronous Output

IF                      OUT         Conditional Asynchronous Output

DEFAULT     NEXT                    Default Next State

DEFAULT                 OUT         Default Asynchronous Output

DEFAULT     NEXT        OUT         Default Synchronous Output

Truth Tables

When logic is best described in tabular form, use the format in the

following decoder example:

FIELD input = [in1..0] ;

FIELD output = [out3..0] ;

TABLE input => output  {

0 => 'b'0001 ;

1 => 'b'0010 ;

2 => 'b'0100 ;

3 => 'b'1000 ;

}

High-Level Equations

output(s).EXT = expression ;

CUPL SYNTAX

�����������

Logical operators

&   =   Logical AND

#   =   Logical OR

$   =   Logical XOR

!   =   Logical NEGATION

Free Form Comment Structure

/*  =   Start Comment

*/  =   End Comment

Variable Extensions

.D            D input of D-type flip-flop

.L            D input of transparent latch

.J            J input of JK-type flip-flop

.K            K input of JK-type flip-flop

.S            S input of SR-type flip-flop

.R            R input of SR�type flip-flop

.T            T input of toggle flip-flop

.DQ           Q output of D-type flip-flop

.LQ           Q output of transparent latch

.AP           Asynchronous preset of flip-flop

.AR           Asynchronous reset of flip-flop

.SP           Synchronous preset of flip-flop

.SR           Synchronous reset of flip-flop

.CK           Programmable clock of flip-flop

.OE           Programmable output enable

.CA           Complement array

.PR           Programmable preload

.CE           CE input of enabled D-CE type flip-flop

.LE           Programmable latch enable

.OBS          Programmable observability of buried nodes

.BYP          Programmable register bypass

.DFB          D registered feedback path selection

.LFB          D latched feedback path selection

.TFB          T registered feedback path selection

.IO           Pin feedback path selection

.INT          Internal feedback path selection

.CKMUX        Clock multiplexor selection

.OEMUX        Tristate multiplexor selection

.IMUX         Input multiplexor selection

.TEC          Technology-dependent fuse selection

Node Declarations

NODE variable_name ;        /* Single node, as when used for      */

/* complement_array: in IFL devices.  */

NODE [variable_list] ;      /* Multiple nodes, as when used for   */

/* buried state bits.                 */

The Distributive Property (from Boolean Algebra)

A & (B # C)   is replaced by   A & B  #  A & C

where & operations are performed before # operations.

DeMorgan's theorem: (from Boolean Algebra)

!(A # B)   is replaced by   !A & !B     also

!(A & B)   is replaced by   !A # !B

NOTE:  This symbology tends to create large numbers of product terms.

Macro Substitution

You can arbitrarily create and define a symbolic name as follows:

MEMREQ = MEMW # MEMR ;

where MEMREQ does not appear as a pin variable name.

MEMREQ can then be used in expressions for other variables.  The value

"MEMW # MEMR" will be substituted wherever MEMREQ is used.

The List Notations

You can represent groups of bits to be equal to a single symbolic name as

follows:

FIELD IOADR = [A7..0] ;

where IOADR can then be used in expressions for [A7..0].

Equality and Address Range

The : operator compares a bit field with a hex constant value or list of

constant values.  For example

IOADR:C3

or

IOADR:[10..3F]

The latter statement will be true for addresses in the range of 10 hex

through 3F hex, inclusive.

NOTE: Hex constant values must contain the proper number of nibbles to

include the most significant bit of the bit-field variable list.

You can also use the : operator to operate on a bit-field variable list, as

follows:

IOADR:&   replaces   A7 & A6 & A5 & A4 & A3 & A2 & A1 & A0

IOADR:#   replaces   A7 # A6 # A5 # A4 # A3 # A2 # A1 # A0

CSIM: THE SIMULATOR

�������������������

CSIM is a stimulus/response function table oriented simulator that compares

each expected response with that which the logic in the associated .PLD

file would produce given the specified stimulus.  The simulator input file

(filename.SI) must contain the same header information as the associated

logic source file (filename.PLD).  Also, CUPL must have been previously run

for the .PLD file with the -A option flag to produce an absolute file

(filename.ABS), and also with the -J flag if you would like CSIM to append

the function table test-vector information to your .JED file to produce a

.JED file with both fuse and testing information.

The general format for the .SI file is:

"Header Information"

ORDER:

var1, var2, . . . , varN ;

VECTORS:

Stimulus pattern1           Response pattern1

Stimulus pattern2           Response pattern2

.                           .

.                           .

.                           .

Stimulus patternN           Response patternN

Within the vector table, inputs are defined with 1 (+5V), and 0 (GND),

while outputs are defined with H (+5V), L (GND), and Z (high impedance).

"Don't Cares" are represented by an X.  An * in the response field causes

the simulator to determine the output according to the logic definition

contained in the .ABS file, which was derived from your .PLD file.

Directives

To run CSIM, select Simulate CUPL file in the main menu.  Select the .SI

file that contains the test vectors for your design.  A list of CSIM option

flags appear.  Choose the needed flags for your design.