Asant PCI Network Driver Installation for Netware 3.x Server

=============================================================

This document describes how to install the Asant PCI Network driver to work

with Netware 3.x Servers.

1.  Install Netware 3.x according to the installation instruction

provided by Novell.

2.  Proceed to complete the Novell NetWare installation and at the Server

Console prompt type:

LOAD A:\NETWARE\NW3.X\SERVER\ASANTPCI FRAME=ETHERNET_802.3

(if the Driver Disk is in drive A:) or

LOAD B:\NETWARE\NW3.X\SERVER\ASANTPCI FRAME=ETHERNET_802.3

(if the Driver Disk is in drive B:).

The file ASANTPCI.LAN will be copied to your hard drive directory

\SERVER.312 by default along with MSM31X.NLM and ETHERTSM.NLM.

NOTES:

======

Below are sample files for STARTUP.NCF and AUTOEXEC.NCF.

C:\SERVER.312 is assumed to be the default Netware Server directory.

STARTUP.NCF:

There is no additional statement added by the driver.

AUTOEXEC.NCF:

LOAD C:\SERVER.312\MSM       ;\SERVER.312 is your Netware directory

LOAD C:\SERVER.312\ETHERTSM

LOAD C:\SERVER.312\ASANTPCI FRAME=ETHERNET_802.3 NAME=E100

BIND IPX TO E100 NET=1000

Note: Rename MSM31X.NLM to MSM.NLM. MSM31X.NLM will still work but when it is

loaded it will issue the following warning message:

"Module name is MSM.NLM not MSM31X.NLM"