Microsoft LAN Manager 2.1 Installation with AsantPCI NDIS Driver

===================================================================

This document describes how to install Microsoft LanManager 2.1 Work-

station with the AsantPCI NDIS 2 driver.

1./ Execute SETUP from the MS LanManager DOS Setup 2.1. After

copying files from the two Microsoft disks, Network Adapter Drivers

is displayed. Select <Other Driver> at the bottom of the screen.

2./ Insert the Asant PCI driver disk and type A:\ (even though the driver

is in \MSLANMAN.DOS\DRIVERS\ETHERNET\ASANTPCI) and press Enter.

3./ The Other Network Adapter Drivers window appears with Asant PCI

Ethernet Adapter as an option. Click OK.

4./ Select the protocol you want to use (NetBeui or TCP/IP) from the

Network Protocols for AsantPCI Ethernet Adapter window.

5./ Select and run LanManager with the Windows environment then proceed to

complete the installation.

6./ Restart the computer.

WARNING: Make sure you use the latest EMM386.EXE. If MS LanManager

uses the old version of EMM386.EXE, it will cause the computer

to hang on reboot.

Sample files for PROTOCOL.INI, SYSTEM.INI, CONFIG.SYS, AUTOEXEC.BAT are

listed below:

PROTOCOL.INI:

=============

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY =NETBEUI

[NETBEUI_XIF]

DRIVERNAME =NETBEUI$

SESSIONS = 6

NCBS = 12

BINDINGS = "ASANTPCI_NIF"

LANABASE = 0

[TCPIP_XIF]

DRIVERNAME =TCPIP$

IPADDRESS = 192 XXX XXX XXX

SUBNETMASK0 = 255 255 255 0

DEFAULTGATEWAY0 =

NBSESSIONS = 6

LOAD = TCPTSR[C],TINYRFC[C],EMSBFR[CR]

UNLOAD = UNLOADT /NOTSR[DC]

BINDINGS = "ASANTPCI_NIF"

LANABASE = 1

[ASANTPCI_NIF]

DRIVERNAME = ASANTPCI$

;SIA_MODE = AUTOSENSE [AUTOSENSE, AUI, TP, TP_FULL_DUPLEX]

SYSTEM.INI;

===========

[BOOT]

......

NETWORK.DRV=LANMAN21.DRV

.....

[BOOT.DESCRIPTION]

NETWORK.DRV=LAN MANAGER 2.1 ENHANCED NETWORK

SECONDNET.DRV=NO ADDITIONAL NETWORK INSTALLED

CONFIG.SYS:

===========

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /I:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHERNET\ASANTPCI\ASANTPCI.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTOCOL\TCPIP\TCPDRV.DOS /I:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTOCOL\TCPIP\NMEM.DOS

AUTOEXEC.BAT:

=============

SET PATH=C:\LANMAN.DOS\NETPROG;%PATH%

C:\LANMAN.DOS\DRIVERS\PROTOCOL\TCPIP\UMB.COM

NET START WORKSTATION

LOAD NETBEUI

LOAD TCPIP

(The last two statements are for the Protocols you want to use.

Therefore you might see more or less depending on the situation)