Asant PCI Network Driver Installation for NetWare 3.x Client

=============================================================

This document describes how to install Novell NetWare 3.x ODI client

software to work with the Asant PCI Network adapter.

1.  Create a directory named NETWARE on your C drive.

2.  Insert the Asant PCI Network Driver Disk into drive A and change to

the subdirectory NETWARE\NW3.X\CLIENT.

3.  Copy the following files from the Driver Disk to the NETWARE directory

on your C drive:

LSL.COM

IPXODI.COM

NETX.COM

NET.CFG

ASANTPCI.COM

4.  Check your CONFIG.SYS file to make sure that the following line appears:

DEVICE=C:\DOS\SETVER.EXE

assuming that \DOS is the directory where you have all your DOS

files including SETVER.EXE.

5.  Remove the Asant PCI Network Driver Disk from drive A.

6.  Restart your computer.

7.  To run NetWare, type the following four commands (in the following sequence)

at the C:\NETWARE prompt:

LSL

ASANTPCI

IPXODI

NETX

8.  Change to the network drive (usually F) and log in to your network.

NOTES:

======

1.  NETX.EXE (dated 930217) will work with DOS 6.0 or lower version only.

You will need NETX.EXE (dated 931117) to work with DOS 6.22.

2.  If you choose to use the Netware Client Install program, follow the

instructions for Netware 4.x Client Installation document.

3.  Remove the line LASTDRIVE=Z from your CONFIG.SYS file, if it exists.

4.  A sample NET.CFG file is shown below:

Link Support

Protocol

Bind ASANTPCI

Link Driver ASANTPCI

Frame    Ethernet_802.3

Protocol IPX 0 Ethernet_802.3

AUI       ;if AUI is the Ethernet connector

;the default is Twisted-pair