19200 : 19,200 Baud Communications Enabler

Version 1.00, 06-Apr-1987

SMG Software

(C) Copyright 1986,1987 by

Steven Georgiades

All Rights Reserved

This  program  is  a utility which,  once  installed  resident  in

memory,   will  allow  you to replace one of the  standard  baud  rates

available  on the PC into 19,200  Baud.   This is useful for performing

direct-link  transfers  between PC's,  or between a PC  and  peripheral

equipment.  I wrote this program so that I could increase my throughput

when  communicating  with  a  PROM programmer.    The  source  code  is

included,  so that you could even patch this program to communicate  at

even higher baud rates (up to 115,200 baud).

To use this program,  enter the program name ("19200"), optionally

followed by the baud rate to be replaced.   For example,  if you  enter

"19200 300",  then  telling your  communications program to run at  300

baud  will  actually cause it to run at 19,200  baud.   If you  do  not

specify  a baud rate,  the default assumed is 110  baud (the  slowest).

You may change your selection at any time after installing the program,

using the same command format.  The program will be able to detect that

it has already been installed,  and will not occupy more memory if  run

multiple times.

Command Format:

19200 [baud]

baud = Baud Rate to Replace.  Permitted Values are: 110, 150,

300, 600, 1200, 2400, 4800, and 9600.

Examples:

19200               (Will replace 110 baud with 19,200 baud)

19200 150           (Will replace 150 baud with 19,200 baud, and

will restore 110 baud)

Revison History:

Version 1.00        Initial Release

19200, Version 1.00                                                      Page 2

This software is distributed as ShareWare.  "ShareWare" means that

you  are granted free license to use and copy this software as much  as

you like, within the following restrictions:

1).  It  is  distributed  in  its  original,   unmodified  form,

including source code and documentation.

2).  No  fee is charged for use,  copying or distribution  (With

the exception of nominal "per disk" copying charges, not to

exceed $6.00 per disk).

3).  The  program  may  not  be included  with  other  goods  or

services  supplied for a fee,  unless written permission to

do so is obtained in advance from the author.

You are not required to register your shareware,  but if you are  using

the  program,   and  find it helpful,  you are  expected  to  register.

Registration fees are as follows:

Full Registration (w/full support) ........ $20.00

Basic Registration (w/basic support) ...... $10.00

Note that you are not LEGALLY obligated to register, but if you use the

software  regularly,  you are MORALLY obligated to do so.   Aside  from

easing your conscience, basic registration will also get you a diskette

containing  the  latest version of the software (unless  you  currently

have  the latest version,  in which case I will send you the next major

revision),  a printed version of the documentation (8-1/2 x 11,  unless

you specify 5-1/2 x 8-1/2), and basic user support,  which means that I

will   work  with  you  to  resolve  problems,   and  will   do   minor

customization.    Full registration will get you everything that  basic

registration  does,  plus full user support,  which includes  telephone

support and all major revisions of the software for two (2)  years from

the date of registration.  Fully registered users will also receive new

SMG  releases  from time to time for evaluation.   In return for  these

evaluations,   the  user  will receive full registration  for  the  new

software at no charge.   When you register,  please specify the name of

the program, and the version number.

Note that no support will be given to unregistered users EXCEPT in

the  case of MAJOR bugs which preclude the unregistered user from using

the program.  I  will then do my best to resolve these problems so that

the user may evaluate the software fully.

19200, Version 1.00                                                      Page 3

This program is provided AS  IS  without  any  warranty,  expressed  or

implied,  including  but  not  limited  to  fitfulness for a particular

purpose.

If  you have any questions or comments about this or any other SMG

program, or if you would like to register any SMG Software, write to me

at the following address:

Steven M. Georgiades

SMG Software

701-H South Hayward Street

Anaheim, CA  92804

Or leave a message for me on the Circuit Board:

The Circuit Board RBBS

SYSOP: Dave Kleinschmidt

(714) 778-0948

300/1200 Baud - No Fee

All  SMG  Software  is  initially released to the  public  through  the

Circuit  Board.   The latest version of any SMG Software can always  be

found there.

A diskette containing all of the latest SMG Software can be obtained by

sending a check or money order for $6.00 to the above address.