LEOCOM 1.0

LEOCOM is a collection of routines used for asyncronous

communications on the IBM PC.

(C) Copyright 1991 by Leo Bicknell, All Rights Reserved.

This version is compiled for Turbo C++, small memory model.

This library is for test and evaluation purposes.  It may

not be used in any Copyrighted programs of any kind, and many not

be used in any programs designed to be sold or liscened.

Registering this library will allow you to recompile for any

memory model, and make modifications to suit your specific needs.

Registration also allows you to use this library royality free in

any commercial or shareware program.  Registered users will also

be elligable for low cost upgrades to the next version.  To

register complete the form in REGISTER.DOC.

If you have any questions, comments, or suggestions I can be

reached on The Apollo III BBS (716) 896-6409 HST, or you can send

a letter US SNAIL to:

Leo Bicknell

46 Foxhunt Lane

East Amherst, NY 14051

Please allow at least 2 days for a responce on the BBS, and

2 weeks for a responce by SNAIL.

---

Now for the actual library.  With this version I am not able

to fully expand on all of the public varables, that will be in

the next version with better documentation.  This should be all

the information you need to get your program up and running

though.

---

void      mclear(void);

Clears the incomming buffer.

---

int       mclose(void);

Shutdown the communication package, a call to MOPEN must

have been prevously made, otherwise undefined results will occur.

---

THIS FUNCTION MUST BE CALLED BEFORE ALL OTHERS!!!!!

int  mopen(int baud, int word, int parity, int stop, int BASE,

int VECTOR, int MASK);

baud    - Baud rate divisor

word    - Word length

parity  - Parity setting

stop    - Stop bits

BASE    - Com port base address

VECTOR  - Interrupt vector number

MASK    - 8259 PIC comtroller mask

These are all provided to allow maximum flexability in the

package.  Definitions for baud, word, parity, and stop can all be

found in LEOCOM.H.  For standard com ports:

COM1            COM2

BASE    0x3f8           0x2f8

VECTOR  12              11

MASK    IM4             IM3

Baud rate divisor can be calculated by the formula:

divisor = (int)(115200l / long_desired_baud_rate);

Word length can be DATA8, DATA7, DATA6, or DATA5,

representing 8, 7, 6, or 5 data bits, respectively.

Parity can be PNONE, PEVEN, or PODD, representing no,

even, and odd parity, respectively.

Stop can be STOP1 or STOP2 representing 1 or 2 stop bits,

respectively.

RETURNS: mopen will return TRUE or FALSE (defined in

LEOCOM.H).  On false return a FATAL error has occured.

---

int       mputs(char *s);

Send a string to the com port.  Port must have been

prevously opened by MOPEN.

RETURNS: Number of charactors sent, or 0 if error.

---

int       mputc(int c);

Send a charactor to the com port.  Port must have been

prevously opened by MOPEN.

RETURNS: TRUE is sent, FALSE if send error.

---

int       mgetc(void);

Get a charactor.  Will automatically return scan code value

on DOORWAY mode keys. Port must have been prevously opened by

MOPEN.

RETURNS: ASCII or SCAN code.

---

void      msdtr(int state);

Set DTR to LOW or HIGH (defined in LEOCOM.H).  Port must

have been prevously opened by MOPEN.

---

void msrts(int state);

Set RTS to LOW or HIGH (defined in LEOCOM.H).  Port must

have been prevously opened by MOPEN.

---

void msbreak(void);

Send a break over the com port.  Port must have been

prevously opened by MOPEN.